Table of Contents

TRAILER MODEL DESIGNATION

NATM Model Trailer User’s Manual

^WARNING

Thuro-Bilt, Inc.

885 Flournoy Valley Rd

Roseburg, OR 97471

www.thuro-bilt.com

541-670-8701

thurobilttrailers@gmail.com

THURO-BILT, INC.

^WARNING

This User’s Manual contains safety information and instructions for your trailer. You must read this manual before loading or towing your trailer. You must follow all safety precautions and instructions.

i

Table of Contents

TRAILER MODEL DESIGNATION

1. GENERAL SAFETY INFORMATION ...........................................................................................................1

1.1. Safety Alert Symbols & Signal Words........................................................................................................1

1.2. Major Hazards............................................................................................................................................1

1.3. Improper Sizing of the Trailer to the Tow Vehicle......................................................................................2

1.4. Driving Too Fast .......................................................................................................................................2

1.5. Failure to Adjust Driving Behavior When Towing a Trailer .......................................................................2

1.6. Trailer Not Properly Coupled to the Hitch..................................................................................................3

1.7. Proper Use of Safety Chains ....................................................................................................................4

1.8. Proper Connection of Breakaway Brake System......................................................................................4

1.9. Matching Trailer & Hitch............................................................................................................................5

1.10. Worn Tires, Loose Wheels & Lug Nuts...................................................................................................5

1.11. Improper Loading....................................................................................................................................6

1.12. Unsafe Load Distribution ........................................................................................................................7

1.13. Shifting Cargo.........................................................................................................................................8

1.14. Inappropriate Cargo................................................................................................................................8

1.15. Inoperable Brakes, Lights or Mirrors.......................................................................................................9

1.16. Hazards From Modifying Your Trailer....................................................................................................10

1.17. Hazards to Horses (Horse Trailer).........................................................................................................10

1.18. Hazards to Livestock (Livestock Trailer)................................................................................................11

1.19. Hazards from Accessories.....................................................................................................................12

1.20. Generator...............................................................................................................................................12

1.21. Shore Power .........................................................................................................................................13

1.22. LP Gas Fuel System ............................................................................................................................14

1.23. Safety Warning Labels on Your Trailer.................................................................................................15

1.24. Trailer Towing Guide ...........................................................................................................................15

1.25. Reporting Safety Defects.....................................................................................................................16

1.26. Safe Trailer Towing Guidelines............................................................................................................17

2. TIRE SAFETY INFORMATION................................................................................................................18

2.1. Steps for Determining Correct Load Limit - Trailer................................................................................18

2.2. Trailers 10,000lbs GVWR or Less ........................................................................................................18

2.3. Trailers Over 10,000lbs GVWR.............................................................................................................19

2.4. Steps for Determining Correct Load Limit - Tow Vehicle.......................................................................19

2.5. Glossary of Tire Terminology................................................................................................................19

2.6. Tire Safety - Everything Rides on it.......................................................................................................23

2.7. Safety First–Basic Tire Maintenance.....................................................................................................23

2.8. Finding Your Vehicle's Recommended Tire Pressure & Load Limits....................................................23

2.9. Understanding Tire Pressure &  Load Limits .......................................................................................24

2.10. Checking Tire Pressure .....................................................................................................................24

2.11. Steps for Maintaining Proper Tire Pressure ......................................................................................24

2.12. Tire Size.............................................................................................................................................25

2.13. Tire Tread ..........................................................................................................................................25

2.14. Tire Balance & Wheel Alignment .......................................................................................................25

2.15. Tire Repair .........................................................................................................................................25

2.16. Tire Fundamentals..............................................................................................................................25

2.17. Information on Passenger Vehicle Tires.............................................................................................25

2.18. UTQGS Information ...........................................................................................................................27

2.19. Additional Information on Light Truck Tires........................................................................................28

2.20. Tire Safety Tips...................................................................................................................................28

3. COUPLING TO THE TOW VEHICLE .....................................................................................................29

3.1. Use an Adequate Tow Vehicle & Hitch ................................................................................................29

3.2. Trailer Information.................................................................................................................................30

3.3. Coupling & Uncoupling the Trailer........................................................................................................32

3.4. Various Coupler Designs......................................................................................................................33

3.5. Trailer with Ball-Hitch Coupler .............................................................................................................34

ii

Table of Contents

TRAILER MODEL DESIGNATION

3.6. Before Coupling the Trailer to the Tow Vehicle ...................................................................................34

3.7. Prepare the Coupler & Hitch................................................................................................................ 35

3.8. Couple the Trailer to the Tow Vehicle.................................................................................................. 36

3.9. Rig the Safety Chains ......................................................................................................................... 37

3.10. Attach & Test Electric Breakaway Brake System.............................................................................. 38

3.11. Connect the Electrical Cables........................................................................................................... 41

3.12. Uncoupling the Ball Hitch Trailer with Tongue Jack.......................................................................... 41

3.13. Trailer with Gooseneck Coupler & Drop-leg Jack ............................................................................ 41

3.14. Prepare the Ball Receiver & Gooseneck Ball................................................................................... 45

3.15. Couple the Trailer to the Tow Vehicle.............................................................................................. 46

3.16. Rig the Safety Chains ...................................................................................................................... 47

3.17. Attach & Test the Breakaway Brake System ................................................................................... 48

3.18. Connect the Electrical Cables.......................................................................................................... 50

3.19. Uncoupling the Gooseneck Trailer with Drop-leg Jack ................................................................... 50

3.20. Trailer with Fifth-wheel Coupler & Drop-leg Jack ............................................................................ 52

3.21. Before Attempting to Tow the Trailer................................................................................................ 52

3.22. Raise the Drop Leg Jack ................................................................................................................. 54

3.23. Attach & Test the Breakaway Brake System ................................................................................... 55

3.24. Connect the Electrical Cables........................................................................................................... 56

3.25. Uncoupling the Fifth-Wheel Trailer with Drop-leg Jack..................................................................... 57

4. LOADING THE TRAILER...................................................................................................................... 59

4.1. Tongue Weight ................................................................................................................................... 59

4.2. Checking Tongue Weight..................................................................................................................... 61

4.3. Securing the Cargo.............................................................................................................................. 62

4.4. Loading Cargo (Enclosed Trailer) ....................................................................................................... 62

4.5. Preparing the Trailer for Loading......................................................................................................... 63

4.6. Loading the Enclosed Trailer............................................................................................................... 63

4.7. Loading Horses (Horse Trailer)............................................................................................................ 64

4.8. Preparing the Horse Trailer for Loading .............................................................................................. 65

4.9. Loading the Horse Trailer .................................................................................................................... 66

4.10. Loading Livestock (Livestock Trailer)................................................................................................. 68

4.11. Preparing the Livestock Trailer for Loading ....................................................................................... 69

4.12. Loading the Livestock Trailer.............................................................................................................. 70

5. CHECKING THE TRAILER BEFORE & DURING EACH TOW ............................................................. 71

5.1. Pre-tow Checklist .................................................................................................................................. 71

5.2. Make Regular Stops.............................................................................................................................. 71

6. BREAKING-IN A NEW TRAILER ........................................................................................................... 72

6.1. Retighten Lug Nuts at First 10,25 & 50 Miles........................................................................................ 72

6.2. Adjust Brake Shoes at First 200 Miles................................................................................................... 72

6.3. Synchronizing the Brake Systems......................................................................................................... 72

7. INSPECTION, SERVICE & MAINTENANCE........................................................................................... 73

7.1. Inspection, Service & Maintenance Summary Charts .......................................................................... 73

7.2. Inspection & Service Instructions.......................................................................................................... 75

7.3. Trailer Structure ................................................................................................................................... 75

7.4. Welds ................................................................................................................................................... 76

7.5. Drop Ramp Torsion Springs ................................................................................................................ 76

7.6. Brake Shoes & Drums ......................................................................................................................... 76

7.7. Manually Adjusting Brake Shoes.......................................................................................................... 76

7.8. Brakes, Electric .................................................................................................................................... 77

7.9. Breakaway Break…………………………………………………………………………………………………77

iii

Table of Contents

TRAILER MODEL DESIGNATION

7.10.Tow Vehicle Operated Electric Brakes……………….......................................................................... 77

7.11. Magnets for all Electric Brakes……………………….......................................................................... 78

7.12. Brakes, Hydraulic (Vacuum, Air or Electric Operated) ..................................................................... 78

7.13. Vacuum-Operated Hydraulic…………………………......................................................................... 78

7.14. Air Pressure-Operated Hydraulic…………………….......................................................................... 78

7.15. Electrical-Operated Hydraulic…………………………........................................................................ 78

7.16. Coupler &  Ball .................................................................................................................................. 78

7.17. Gooseneck .........................................................................................................................................79

7.18. Fifth Wheel Kingpin ........................................................................................................................... 79

7.19. Landing Leg or Jack........................................................................................................................... 79

7.20. Lights & Signals .................................................................................................................................79

7.21. Accessory Battery...............................................................................................................................80

7.22. Tires.....................................................................................................................................................80

7.23. Wheel Rims .........................................................................................................................................81

7.24. Wheels, Bearings &  Lug Nuts ............................................................................................................81

7.25. Unsealed Bearings (Hubs) ..................................................................................................................81

7.26. Lug Nuts (Bolts) ..................................................................................................................................82

8. WARNING STICKERS ON THE TRAILER............................................................................................. 84

iv

General Safety Information

- GENERAL SAFETY INFORMATION

1.1. SAFETY ALERT SYMBOLS & SIGNAL WORDS

An Owner’s Manual that provides general trailer information cannot cover all of the specific details necessary

for the proper combination of every trailer, tow vehicle and hitch. Therefore, you must read, understand and

follow the instructions given by the tow vehicle and trailer hitch manufacturers, as well as the instructions in this

manual.

Our trailers are built with components produced by various manufacturers. Some of these items have separate

instruction manuals. Where this manual indicates that you should read another manual, and you do not have

that manual, call Thuro-Bilt, Inc at 541-670-8701 for a free copy.

The safety information in this manual is denoted by the safety alert symbol: ^

The level of risk is indicated by the following signal words.

^ DANGER

DANGER – Immediate hazards which WILL result in severe personal injury or death if the warning is ignored.

^WARNING

^WARNING

WARNING – Hazards or unsafe practices which COULD result in severe personal injury or death if the warning is ignored.

^ CAUTION

CAUTION – Hazards or unsafe practices which could result in minor or moderate injury if the warning is ignored.

^NOTICE

NOTICE – Practices that could result in damage to the trailer or other property.

1.2. MAJOR HAZARDS

Loss of control of the trailer or trailer/tow vehicle combination can result in death or serious injury. The most

common causes for loss of control of the trailer are:

• Improper sizing the trailer for the tow vehicle, or vice versa.

• Excessive Speed: Driving too fast for the conditions.

• Failure to adjust driving behavior when towing a trailer.

1

General Safety Information

• Overloading and/or improper weight distribution.

• Improper or mis-coupling of the trailer to the hitch.

• Improper braking and steering under sway conditions.

• Not maintaining proper tire pressure.

• Not keeping lug nuts tight.

1.3. IMPROPER SIZING OF THE TRAILER TO THE TOW VEHICLE

Trailers that weigh too much for the towing vehicle can cause stability problems, which can lead to death or

serious injury. Furthermore, the additional strain put on the engine and drive-train may lead to serious tow

vehicle maintenance problems. For these reasons the maximum towing capacity of your towing vehicle should

not be exceeded. The towing capacity of your tow vehicle, in terms of maximum Gross Trailer Weight (GTW)

and maximum Gross Combined Weight Rating (GCWR) can be found in the tow vehicles Owner’s Manual.

^ DANGER

Use of a tow vehicle with a towing capacity less than the Gross Vehicle Weight Rating of the trailer can result in loss of control, and may lead to death or serious injury. Be sure your hitch and tow vehicle are rated for the Gross Vehicle Weight Rating of your trailer.

1.4. DRIVING TOO FAST

With ideal road conditions, the maximum recommended speed for safely towing a trailer is 60 mph. If you drive

too fast, the trailer is more likely to sway, thus increasing the possibility for loss of control. Also your tires may overheat, thus increasing the possibility of a blowout.

^WARNING

Driving too fast for conditions can result in loss of control and cause death or serious injury. Decrease your speed when going downhill or as road, weather, and lighting conditions deteriorate.

1.5. FAILURE TO ADJUST DRIVING BEHAVIOR WHEN TOWING A TRAILER

When towing a trailer, you will have decreased acceleration, increased stopping distance, and increased

turning radius (which means you must make wider turns to keep from hitting curbs, vehicles, and anything else

that is on the inside corner). Furthermore the trailer will change the handling characteristics of your towing

vehicle, making it more sensitive to steering inputs and more likely to be pushed around in windy conditions or

when being passed by large vehicles. In addition, you will need a longer distance to pass, due to slower

acceleration and increased length. With these caveats in mind:

• Be alert for slippery conditions. You are more likely to be affected by slippery road surfaces when

driving a tow vehicle with a trailer, than driving a tow vehicle without a trailer.

• Anticipate the trailer “swaying.” Swaying can be caused by excessive steering, wind gusts, roadway

edges, or by the trailer reaction to the pressure wave created by passing trucks and busses.

• When encountering trailer sway take your foot off the gas, and steer as little as possible in order to stay

on the road. Use small “trim-like” steering adjustments. Do not attempt to steer out of the sway; you’ll

2

General Safety Information

only make it worse. Also do not apply the tow vehicle brakes to correct trailer swaying. On the other

hand, application of the trailer brakes alone will tend to straighten out the combination, especially when

going downhill.

• Check rearview mirrors frequently to observe the trailer and traffic.

• Use lower gear when driving down steep or long grades. Use the engine and transmission as a brake.

Do not ride the brakes, as they can overheat and become ineffective.

• Be aware of your trailer height, especially when approaching bridges, roofed areas and around trees.

1.6. TRAILER NOT PROPERLY COUPLED TO THE HITCH

It is critical that the trailer be securely coupled to the hitch ball, and that the safety chains and emergency

break-away brake cable is correctly attached. Uncoupling may result in death or serious injury to you and to

others.

^WARNING

Proper selection and condition of the coupler and hitch are essential to safely towing your trailer. A loss of coupling may result in death or serious injury. • Be sure the maximum load rating of the hitch on the tow vehicle is equal to or greater than the maximum load rating of the coupler on the trailer. • Be sure the hitch ball size matches the coupler size. • Observe the hitch for wear, corrosion and cracks before coupling. Replace worn, corroded or cracked hitch components before coupling the trailer to the tow vehicle. • Be sure the hitch components are tight before coupling the trailer to the tow vehicle.

3

General Safety Information

^

^WARNING

An improperly coupled trailer can result in death or serious injury. Do not move the trailer until: • The coupler is secured and locked to hitch ball; • The safety chains are secured to the tow vehicle; and • The trailer jack(s) are fully retracted. Do not tow the trailer on the road until: • Tires and wheels are checked; • The trailer brakes are checked; • The breakaway switch is connected to the tow vehicle; • The load is secured to the trailer; and • The trailer lights are connected and checked.

1.7. PROPER USE OF SAFETY CHAINS

If your trailer comes loose from the hitch for any reason, we have provided safety chains so that control of the

trailer can still be maintained

^WARNING

Improper rigging of the safety chains can result in loss of control of the trailer and tow vehicle, leading to death or serious injury, if the trailer uncouples from the tow vehicle. • Fasten chains to frame of tow vehicle. Do not fasten chains to any part of the hitch unless the hitch has holes or loops specifically for that purpose. • Cross chains underneath hitch and coupler with enough slack to permit turning and to hold tongue up, if the trailer comes loose.

1.8.  PROPER CONNECTION OF BREAKAWAY BRAKE SYSTEM

If equipped with brakes your trailer will be equipped with a breakaway brake system that can apply the brakes

on your trailer if your trailer comes loose from the hitch ball for any reason.

^WARNING

4

General Safety Information

^

^WARNING

An ineffective or inoperative breakaway brake system can result in a runaway trailer, leading to death or serious injury, if the coupler or hitch fails. The breakaway cable must be connected to the tow vehicle; and NOT to any part of the hitch. Before towing the trailer, test the function of the breakaway brake system. If the breakaway brake system is not working, do not tow the trailer; have it serviced or repaired.

1.9. MATCHING TRAILER & HITCH

^ DANGER

Use of a hitch with a load rating less than the load rating of the trailer can result in loss of control and may lead to death or serious injury.

1.10. WORN TIRES, LOOSE WHEELS & LUG NUTS

Just as with your tow vehicle the trailer tires and wheels are important safety items. Therefore, it is essential to

inspect the trailer tires before each tow.

If a tire has a bald spot, bulge, cut, cracks, or is showing any cords, replace the tire before towing. If a tire has

uneven tread wear, take the trailer to a dealer service center for diagnosis. Uneven tread wear can be caused

by tire imbalance, axle misalignment or incorrect inflation.

Tires with too little tread will not provide adequate frictional forces on wet roadways and can result in loss of

control, leading to death or serious injury.

Improper tire pressure causes increased tire wear and may reduce trailer stability, which can result in a tire

blowout or possible loss of control. Therefore, before each tow you must also check the tire pressure.

Remember, the proper tire pressure is listed on the Certification / VIN label, normally mounted on front left side

of the trailer, and should be checked when tires are cold. Allow 3 hours cool-down after driving as much as 1

mile at 40 mph before checking tire pressure.

^WARNING

Improper tire pressure can result in a blowout and loss of control, which can lead to death or serious injury. Be sure tires are inflated to pressure indicated on the Certification/VIN label before towing trailer.

The tightness of the lug nuts is very important in keeping the wheels properly seated to the hub.

Before each tow, check to make sure they are tight.

5

General Safety Information

^WA

^WARNING

Metal creep between the wheel rim and hub may cause lug nuts to loosen and could result in a wheel coming off, leading to death or serious injury. Tighten lug nuts before each tow.

The proper tightness (torque) for lug nuts is listed in Section 7.26 in the “Inspection and Service & Maintenance”

chapter of this manual. Use a torque wrench to tighten the lug nuts, use the crisscross star

pattern. If you do not have a torque wrench, use a lug wrench (from your tow vehicle) and tighten

the nuts as much as you can. At the first opportunity, have a service garage or trailer dealer tighten the lug

nuts to the proper torque.

Lug nuts are also prone to loosen after first being assembled. When driving a new trailer (or after wheels have

been remounted), check to make sure they are tight after the first 10, 25 and 50 miles of driving and before

each tow thereafter.

Failure to perform this check can result in a wheel separating from the trailer and a crash, leading to death or

serious injury.

^WARNING

Lug nuts are prone to loosen after initial installation, which can lead to death or serious injury. Check lug nuts for tightness on a new trailer or when wheel(s) have been remounted after the first 10, 25 and 50 miles of driving.

^WARNING

Improper lug nut torque can cause a wheel separating from the trailer, leading to death or serious injury. Be sure lug nuts are tight before each tow.

1.11. IMPROPER LOADING

The total weight of the load you put in or on the trailer, plus the empty weight of the trailer itself, must not

exceed the trailer's Gross Vehicle Weight Rating (GVWR). If you do not know the empty weight of the trailer

plus the cargo weight, you must weigh the loaded trailer at a commercial scale. In addition, you must distribute

the load in the trailer such that the load on any axle does not exceed the Gross Axle Weight Rating (GAWR). If

your trailer is equipped with a Tire & Loading Information Placard, mounted next to the Certification / VIN label,

the cargo capacity weight stated on that placard is only a close estimate. The GVWR and GAWR’s are listed

on the Certification / VIN label mounted on the front left side of the trailer.

6

General Safety Information

^WARNING

An overloaded trailer can result in loss of control of the trailer, leading to death or serious injury. Do not exceed the trailer Gross Vehicle Weight Rating (GVWR) or an axle Gross Axle Weight Rating (GAWR). Do not load a trailer so that the weight on any tire exceeds its rating.

1.12. UNSAFE LOAD DISTRIBUTION

Improper front / rear load distribution can lead to poor trailer sway stability or poor tow vehicle handling. Poor

trailer sway stability results from tongue weights that are too low, and poor tow vehicle stability results from

tongue weights that are too high. Refer to Chapter heading “Loading the Trailer” for more information.

In the table below, the second column shows the rule of thumb percentage of total weight of the trailer plus its

cargo (Gross Trailer Weight, or “GTW”) that should appear on the tongue of the trailer. For example, a trailer

with a gooseneck hitch, with a loaded weight of 12,000 pounds, should have 20-25% of 12,000 pounds (2400-

3000 lbs.) on the gooseneck. A dump trailer will have the proper weight distribution if the load is evenly

distributed in the dump bed. For non-flowable (discrete) loads locate the load such as to provide the proper

tongue weight.

After loading, be sure to check that none of the axles are overloaded.

| Tongue Weight as a Percentage of Loaded Trailer Weight |  |
| --- | --- |
| Type of Hitch | Percentage |
| Ball Hitch (or Bumper Hitch) | 10–15%for large trailers 6-10% for smaller utility and cargo trailers 4-6% for boat trailers |
| Gooseneck or Fifth Wheel Hitch | 20–25% |

The numbers quoted above are for example purposes only and should be tailored to the specific trailer.

For questions regarding the actual percent of tongue weight for the trailer, check with the manufacturer

for specifics.

Uneven left / right load distribution can cause tire, wheel, axle or structural failure. Be sure your trailer is evenly

loaded left / right. Towing stability also depends on keeping the center of gravity as low as possible.

7

General Safety Information

^WARNING

Improper tongue weight (load distribution) can result in loss of control of the trailer, leading to death or serious injury. Make certain that tongue weight is within the allowable range. Be sure to: • Distribute the load front-to-rear to provide proper tongue weight (see chart above). For dump trailers a flowable load should be evenly distributed throughout the bed. • Distribute the load evenly, right and left. • Keep the center of gravity low.

1.13. SHIFTING CARGO

Since the trailer “ride” can be bumpy and rough, you must secure your cargo so that it does not shift while the

trailer is being towed.

^WARNING

Shifting cargo can result in loss of control of the trailer, and can lead to death or serious injury. Tie down all loads with proper sized fasteners, ropes, straps, etc.

If the door latch is equipped with a catch that has a hole for a linchpin, use a linchpin to prevent the door latch from opening.

^WARNING

If the door opens, your cargo may be ejected onto the road, resulting in death or serious injury to other drivers. Always secure the door latch after closing. Place a linchpin in the catch.

1.14. INAPPROPRIATE CARGO

Your trailer may be designed for specific cargo, for example, only for horses. If your trailer is designed for

specific cargo, only carry that cargo in the trailer. A utility trailer must not be used to carry certain items, such

as people, containers of hazardous substances or containers of flammable substances.

8

General Safety Information

^WARNING

Do not transport people inside the trailer, even if it has living quarters. The transport of people puts their lives at risk and may be illegal.

^

^WARNING

Do not transport flammable, explosive, poisonous or other dangerous materials in your trailer. Exceptions: • Fuel in the tanks of vehicles that are being towed • Fuel stored in proper containers used in trailer living quarters for cooking • Fuel stored in the tank of an on-board generator.

1.15. INOPERABLE BRAKES, LIGHTS OR MIRRORS

Be sure that the electric brakes and all of the lights on your trailer are functioning properly before towing your

trailer. Electric brakes and lights on a trailer are controlled via a connection to the tow vehicle, generally a

multi-pin electrical connector. Check the trailer tail lights by turning on your tow vehicle headlights. Check the

trailer brake lights by having someone step on the tow vehicle brake pedal while you look at trailer lights. Do

the same thing to check the turn signal lights.

If your trailer has electric brakes, your tow vehicle will have an electric brake controller that sends power to the

trailer brakes. Before towing the trailer on the road, you must operate the brake controller while trying to pull

the trailer in order to confirm that the electric brakes operate. While towing the trailer at less than 5 mph,

manually operate the electric brake controller in the tow vehicle cab. You should feel the operation of the trailer

brakes.

^WARNING

Improper electrical connection between the tow vehicle and the trailer will result in inoperable lights and electric brakes, and can lead to collision. Before each tow: • Check that the taillights, brake lights and turn signals work • Check that the electric brakes work by operating the brake controller inside the tow vehicle.

If your trailer has hydraulic “surge” brakes, pull the emergency break-away brake lanyard to check the

operation of the surge mechanism.

9

General Safety Information

Standard mirrors usually do not provide adequate visibility for viewing traffic to the sides and rear a towed

trailer. You must provide mirrors that allow you to safely observe approaching traffic.

1.16. HAZARDS FROM MODIFYING YOUR TRAILER

Essential safety items can be damaged by altering your trailer. Even simply driving a nail or screw to hang

something can damage an electrical circuit, LP gas line or other feature of the trailer.

Before making any alteration to your trailer, contact your dealer and describe the alteration you are contemplating. Alteration of the trailer structure or modification of mechanical, electrical, plumbing, heating or other systems on your trailer must be performed only by qualified technicians who are familiar with the system as installed on your trailer.

1.17. HAZARDS TO HORSES (HORSE TRAILER)

Before hauling a horse, you must be aware of its temperament.

The layout of a horse trailer is designed to safely contain your horse. The trailer is equipped with stall dividers

and tie rings to secure the horse. Restraining a horse without using a combination of a tie-strap and stall

dividers may result in serious injury or death to the horse.

Before loading your horse, inspect the interior of the horse trailer to insure that no hazards are present. Read

the “Loading the Horse Trailer” sections 4.7, 4.8, & 4.9 of this manual for specific instructions regarding trailering of

horses.

^

^WARNING

When a horse is frightened, it is capable of inflicting serious injury or death to a human handler. Know your horse’s temperament before attempting to trailer it. Handling a horse that is not trailer-acclimated may result in injury or death, or damage to your trailer. Do not haul an unbroken horse in this trailer. Horses must have a halter.

^ CAUTION

Failure to secure a horse using a tie strap may result in its serious injury or death.

10

General Safety Information

^ CAUTION

The trailer interior may contain hazards to a horse that can result in its serious injury or death. Before loading a horse, inspect the trailer interior and adjust or repair all loose and protruding features such as handles, loose or broken parts of the trailer, etc. Before towing trailer: • Lock all stall dividers. • Be sure all saddles, tack and equipment, as well as horse(s), are prevented from being thrown about.

^ CAUTION

Hauling a horse in a livestock trailer may result in its serious injury or death. Do not carry a horse in a livestock trailer. Use a trailer designed to carry horses.

1.18. HAZARDS TO LIVESTOCK (LIVESTOCK TRAILER)

A livestock trailer is designed for the safe transport of livestock, other than horses. It is not equipped for

hauling horses.

Before loading your livestock, inspect the interior of the livestock trailer to insure that no hazards are present.

Read sections 4.10, 4.11 & 4.12 “Loading Livestock (Livestock Trailer)” for specific instructions regarding trailering of

livestock other than horses.

^WARNING

^WARNING

Large animals are capable of inflicting serious injury or death to a human handler. Know your animals’ temperament before attempting to trailer them.

^ CAUTION

Hauling a horse in a livestock trailer may result in its serious injury or death. Do not carry a horse in a livestock trailer. Use a trailer designed to carry horses.

11

General Safety Information

1.19. HAZARDS FROM ACCESSORIES

Read and follow all of these instructions before operating the accessories. The major hazards from some of

these accessories are:

1.20. GENERATOR

If your trailer is equipped with a gasoline or diesel generator, you must have and follow the generator

manufacturer’s instructions. You must also have one or more carbon monoxide detectors in the trailers

accommodation spaces.

Carbon Monoxide is an odorless gas that can cause death. Be certain exhaust from a running generator does

not accumulate in or around your trailer, by situations such as:

• Being drawn in by fans or ventilators operated in a trailer;

• Prevailing wind;

• Being trapped between your trailer and other trailers, vehicles or buildings; or

• Being trapped between your trailer and, or in a snow bank, or other nearby objects

^

^WARNING

Operating gasoline and diesel generators can lead to death or serious injury by: • Carbon Monoxide • Fire and Explosion • Electrocution Have a working carbon monoxide detector in The accommodation spaces before operating a generator. Do not refuel a running generator or refuel near ignition sources.

12

General Safety Information

1.21. SHORE POWER

“Shore Power” is the name given to connecting your trailer to a source of electrical power using an extension

cord specifically designed for that purpose.

^WARNING

Shore power poses a risk of death due to electrocution or fire • Always use an electrical cord specifically designed for shore power connection. Never use an ordinary extension cord. • Always connect the electrical cord to a grounded source of shore power. • Do not remove the “third prong” from the shore power plug. • Connect only to source of proper voltage. • Make certain polarity is correct. • Do not overload electrical circuits. • Always replace fuses or circuit breakers with correct rating.

13

General Safety Information

1.22 LP GAS FUEL SYSTEM

^ DANGER

You can die or be brain damaged by Carbon Monoxide. Make certain the exhaust from LP appliances is directed to the outdoors. Have a working carbon monoxide detector in the accommodation spaces of your trailer before operating any LP gas appliance. Do not operate portable grills or stoves inside the trailer.

^

^WARNING

Risk of death due to fire or explosion. Only connect an LP gas system to a supply of LP gas, NOT natural gas. Do not store LP gas tanks inside the trailer. Only fill an LP gas tank 80% full. Only fill the tank with LP gas (butane or propane). Overfilled tanks can release gas and cause an explosion.

^

^WARNING

Risk of fire or explosion If LP gas is detected (by smell or by the LP gas detector): • Do not touch electrical switches • Extinguish flames and pilot lights • Open doors for ventilation • Shut off LP gas supply at the LP tank • Leave the area until odor clears Correct the source of LP gas leakage before using LP appliances. Do not use a flame to locate the source of an LP gas leak.

14

General Safety Information

W

^WARNING

Risk of fire or explosion Never use a flame, heat lamp or hair dryer to thaw an LP gas regulator. Use an incandescent light bulb. Do not remove the regulator cover or attempt to service the LP gas regulator.

1.23. SAFETY WARNING LABELS ON YOUR TRAILER

^

^WARNING

To protect you and others against death or serious injury, all of the labels shown above must be on the trailer and must be legible. If any of these labels are missing or cannot be read, call Thuro-Bilt, Inc. at 541-670-8701 for free replacement labels. You will need to provide us with the number shown at the bottom of the label(s) in order for us to send the correct one(s).

1.24. TRAILER TOWING GUIDE

Driving a vehicle with a trailer in tow is vastly different from driving the same vehicle without a trailer in tow.

Acceleration, maneuverability and braking are all diminished with a trailer in tow. It takes longer to get up to

speed; you need more room to turn and pass, and more distance to stop when towing a trailer. You will need

to spend time adjusting to the different feel and maneuverability of the tow vehicle with a loaded trailer.

Because of the significant differences in all aspects of maneuverability when towing a trailer, the hazards and

risks of injury are also much greater than when driving without a trailer. You are responsible for keeping your

vehicle and trailer in control, and for all the damage that is caused if you lose control of your vehicle and trailer.

As you did when learning to drive an automobile, find an open area with little or no traffic for your first practice

trailering. Of course, before you start towing the trailer, you must follow all of the instructions for inspection,

testing, loading and coupling. Also, before you start towing, adjust the mirrors so you can see the trailer as well

as the area to the rear of it.

Drive slowly at first, 5 mph or so, and turn the wheel to get the feel of how the tow vehicle and trailer

combination responds. Next, make some right and left hand turns. Watch in your side mirrors to see how the

trailer follows the tow vehicle. Turning with a trailer attached requires more room.

Stop the rig a few times from speeds no greater than 10 mph. If your trailer is equipped with brakes, try using

different combinations of trailer/electric brake and tow vehicle brake. Note the effect that the trailer brakes

have when they are the only brakes used. When properly adjusted, the trailer brakes will come on just before

the tow vehicle brakes.

It will take practice to learn how to back up a tow vehicle with a trailer attached. Take it slow. Before backing

up, get out of the tow vehicle and look behind the trailer to make sure that there are no obstacles. Some drivers

place their hands at the bottom of the steering wheel, and while the tow vehicle is in reverse, “think” of the

hands as being on the top of the wheel. When the hands move to the right (counter-clockwise, as you would

do to turn the tow vehicle to the left when moving forward), the rear of the trailer moves to the right.

Conversely, rotating the steering wheel clockwise with your hands at the bottom of the wheel will move the rear

15

General Safety Information

of the trailer to the left, while backing up. If you are towing a bumper hitch rig, be careful not to allow the trailer

to turn too much, because will hit the rear of the tow vehicle. To straighten the rig, either pull forward, or turn

the steering wheel in the opposite direction.

1.25. REPORTING SAFETY DEFECTS

If you believe that your vehicle has a defect that could cause a crash or could cause injury or death, you should

immediately inform the National Highway Traffic Safety Administration (NHTSA) in addition to notifying

Thuro-Bilt, Inc. Inc at 541-670-8701.

If NHTSA receives similar complaints, it may open an investigation, and if it finds that a safety defect exists in

a group of vehicles, it may order a recall and remedy campaign. However, NHTSA cannot become involved

in individual problems between you, your dealer, or Thuro-Bilt, Inc.

To contact NHTSA, you may either call the Vehicle Safety Hotline toll free 1-888-327-4236

(TTY: 1-800-424-9153), go to http://www.safercar.gov; or write to:

Administrator

NHTSA Headquarters

1200 New Jersey Ave SE

West Building

Washington, DC 20590

You can also obtain other information about motor vehicle safety from http://www.safercar.gov/.

16

General Safety Information

1.26. SAFE TRAILER TOWING GUIDELINES

• Recheck the load tie downs to make sure the load will not shift during towing.

• Before towing, check coupling, safety chain, safety brake, tires, wheels and lights.

• Check the lug nuts or bolts for tightness.

• Check coupler tightness after towing 50 miles.

• Adjust the brake controller to engage the trailer brakes before the tow vehicle brakes. Follow the

instructions given with the brake controller manufacturer’s literature.

• Use your mirrors to verify that you have room to change lanes or pull into traffic.

• Use your turn signals well in advance.

• Allow plenty of stopping space for your trailer and tow vehicle.

• Do not drive so fast that the trailer begins to sway due to speed. Generally never drive faster than 60

m.p.h.

• Allow plenty of room for passing. A rule of thumb is that the passing distance with a trailer is 4 times

the passing distance without a trailer.

• Shift your automatic transmission into a lower gear for city driving.

• Use lower gears for climbing and descending grades.

• Do not ride the brakes while descending grades, they may get so hot that they stop working. Then you

will potentially have a runaway tow vehicle and trailer.

• To conserve fuel, don't use full throttle to climb a hill. Instead, build speed on the approach.

• Slow down for bumps in the road. Take your foot off the brake when crossing the bump.

• Do not brake while in a curve unless absolutely necessary. Instead, slow down before you enter the

curve.

• Do not apply the tow vehicle brakes to correct extreme trailer swaying. Instead, lightly apply the trailer

brakes with the hand controller.

• Make regular stops, about once each hour. Confirm that

The coupler is secure to the hitch and is locked,

Electrical connectors are made,

There is appropriate slack in the safety chains,

There is appropriate slack in the breakaway switch pullpin cable,

The tires are not visibly low on pressure, and

The cargo is secure and in good condition.

17

Tire Safety Information

2. TIRE SAFETY INFORMATION

This portion of the User’s Manual contains tire safety information as required by 49 CFR 575.6.

Section 2.1. contains: “Steps for Determining Correct Load Limit - Trailer”.

Section 2.4. contains: “Steps for Determining Correct Load Limit – Tow Vehicle”.

Section 2.5. contains: a Glossary of Tire Terminology, including “cold inflation pressure”, “maximum inflation

pressure”, “recommended inflation pressure”, and other non-technical terms.

Section 2.6. contains: information from the NHTSA brochure entitled “Tire Safety – Everything Rides On It”.

This brochure describes the following items;

• Tire labeling, including a description and explanation of each marking on the tires, and information

about the DOT Tire Identification Number (TN).

• Recommended tire inflation pressure, including a description and explanation of:

A. Cold inflation pressure.

B. Vehicle Placard and location on the vehicle.

C. Adverse safety consequences of under inflation (including tire failure).

D. Measuring and adjusting air pressure for proper inflation.

• Tire Care, including maintenance and safety practices.

• Vehicle load limits, including a description and explanation of the following items:

A. Locating and understanding the load limit information, total load capacity, and cargo capacity.

B. Calculating total and cargo capacities with varying seating configurations including quantitative examples

showing / illustrating how the vehicles cargo and luggage capacity decreases as combined number and size

of occupants’ increases. This item is also discussed in Section 3.

C. Determining compatibility of tire and vehicle load capabilities.

D. Adverse safety consequences of overloading on handling and stopping on tires.

2.1. STEPS FOR DETERMINING CORRECT LOAD LIMIT – TRAILER

Tire and Loading Information Placard – Figure 1-1

2.2. TRAILERS 10,000 POUNDS GVWR OR LESS

1. Locate the statement, “The weight of cargo should never exceed XXX kg or XXX lbs.,” on your vehicles

placard. See figure 1-1.

2. This figure equals the available amount of cargo and luggage load capacity.

3. Determine the combined weight of luggage and cargo being loaded on the vehicle. That weight may not

safely exceed the available cargo and luggage load capacity.

The trailer’s placard refers to the Tire Information Placard attached adjacent to or near the trailer’s VIN

(Certification) label at the left front of the trailer.

18

Tire Safety Information

Note: The following calculations in Sections 2.1. and 2.2 are not required by the government. For the

purpose of completeness, NATM has included these statements.

2.3. TRAILERS OVER 10,000 POUNDS GVWR (NOTE: THESE TRAILERS ARE NOT REQUIRED TO HAVE A TIRE

INFORMATION PLACARD ON THE VEHICLE)

1. Determine the empty weight of your trailer by weighing the trailer using a public scale or other means. This

step does not have to be repeated.

2. Locate the GVWR (Gross Vehicle Weight Rating) of the trailer on your trailer’s VIN (Certification) label.

3. Subtract the empty weight of your trailer from the GVWR stated on the VIN label. That weight is the

maximum available cargo capacity of the trailer and may not be safely exceeded.

2.4. STEPS FOR DETERMINING CORRECT LOAD LIMIT – TOW VEHICLE

1. Locate the statement, “The combined weight of occupants and cargo should never exceed XXX lbs.,” on

your vehicle’s placard.

2. Determine the combined weight of the driver and passengers who will be riding in your vehicle.

3. Subtract the combined weight of the driver and passengers from XXX kilograms or XXX pounds.

4. The resulting figure equals the available amount of cargo and luggage capacity. For example, if the “XXX”

amount equals 1400 lbs. and there will be five 150 lb. passengers in your vehicle, the amount of available

cargo and luggage capacity is 650 lbs. (1400-750 (5 x 150) = 650 lbs.).

5. Determine the combined weight of luggage and cargo being loaded on the vehicle. That weight may not

safely exceed the available cargo and luggage capacity calculated in Step # 4.

6. If your vehicle will be towing a trailer, load from your trailer will be transferred to your vehicle. Consult the

tow vehicle’s manual to determine how this weight transfer reduces the available cargo and luggage

capacity of your vehicle.

2.5. GLOSSARY OF TIRE TERMINOLOGY

Accessory weight

The combined weight (in excess of those standard items which may be replaced) of automatic transmission,

power steering, power brakes, power windows, power seats, radio and heater, to the extent that these items

are available as factory-installed equipment (whether installed or not).

Bead

The part of the tire that is made of steel wires, wrapped or reinforced by ply cords and that is shaped to fit the

rim.

Bead separation

This is the breakdown of the bond between components in the bead.

Bias ply tire

A pneumatic tire in which the ply cords that extend to the beads are laid at alternate angles substantially less

than 90 degrees to the centerline of the tread.

Carcass

The tire structure, except tread and sidewall rubber which, when inflated, bears the load.

Chunking

The breaking away of pieces of the tread or sidewall.

Cold inflation pressure

The pressure in the tire before you drive.

19

Tire Safety Information

Cord

The strands forming the plies in the tire.

Cord separation

The parting of cords from adjacent rubber compounds.

Cracking

Any parting within the tread, sidewall, or inner liner of the tire extending to cord material.

CT

A pneumatic tire with an inverted flange tire and rim system in which the rim is designed with rim flanges

pointed radially inward and the tire is designed to fit on the underside of the rim in a manner that encloses the

rim flanges inside the air cavity of the tire.

Curb weight

The weight of a motor vehicle with standard equipment including the maximum capacity of fuel, oil, and coolant,

and, if so equipped, air conditioning and additional weight optional engine.

Extra load tire

A tire designed to operate at higher loads and at higher inflation pressures than the corresponding standard

tire.

Groove

The space between two adjacent tread ribs.

Innerliner

The layer(s) forming the inside surface of a tubeless tire that contains the inflating medium within the tire.

Innerliner separation

The parting of the innerliner from cord material in the carcass.

Intended outboard sidewall

The sidewall that contains a white-wall, bears white lettering or bears manufacturer, brand, and/or model name

molding that is higher or deeper than the same molding on the other sidewall of the tire or the outward facing

sidewall of an asymmetrical tire that has a particular side that must always face outward when mounted on a

vehicle.

Light truck (LT) tire

A tire designated by its manufacturer as primarily intended for use on lightweight trucks or multipurpose

passenger vehicles.

Load rating

The maximum load that a tire is rated to carry for a given inflation pressure.

Maximum load rating

The load rating for a tire at the maximum permissible inflation pressure for that tire.

Maximum permissible inflation pressure

The maximum cold inflation pressure to which a tire may be inflated.

Maximum loaded vehicle weight

The sum of curb weight, accessory weight, vehicle capacity weight, and production options weight.

Measuring rim

The rim on which a tire is fitted for physical dimension requirements.

20

Tire Safety Information

Non-pneumatic rim

A mechanical device which, when a non-pneumatic tire assembly incorporates a wheel, supports the tire, and

attaches, either integrally or separably, to the wheel center member and upon which the tire is attached.

Non-pneumatic spare tire assembly

A non-pneumatic tire assembly intended for temporary use in place of one of the pneumatic tires and rims that

are fitted to a passenger car in compliance with the requirements of this standard.

Non-pneumatic tire

A mechanical device which transmits, either directly or through a wheel or wheel center member, the vertical

load and tractive forces from the roadway to the vehicle, generates the tractive forces that provide the

directional control of the vehicle and does not rely on the containment of any gas or fluid for providing those

functions.

Non-pneumatic tire assembly

A non-pneumatic tire, alone or in combination with a wheel or wheel center member, which can be mounted on

a vehicle.

Normal occupant weight

This means 68 kilograms (150 lbs.) times the number of occupants.

Occupant distribution

The distribution of occupants in a vehicle.

Open splice

Any parting at any junction of tread, sidewall, or innerliner that extends to cord material.

Outer diameter

The overall diameter of an inflated new tire.

Overall width

The linear distance between the exteriors of the sidewalls of an inflated tire, including elevations due to

labeling, decorations, or protective bands or ribs.

Ply

A layer of rubber-coated parallel cords.

Ply separation

A parting of rubber compound between adjacent plies.

Pneumatic tire

A mechanical device made of rubber, chemicals, fabric and steel or other materials, that, when mounted on an

automotive wheel, provides the traction and contains the gas or fluid that sustains the load.

Production options weight

The combined weight of those installed regular production options weighing over 2.3 kilograms (5 lbs.) in

excess of those standard items which they replace, not previously considered in curb weight or accessory

weight, including heavy duty brakes, ride levelers, roof rack, heavy duty battery, and special trim.

Radial ply tire

A pneumatic tire in which the ply cords that extend to the beads are laid at substantially 90 degrees to the

centerline of the tread.

Recommended inflation pressure

This is the inflation pressure provided by the vehicle manufacturer on the Tire Information label and on the

Certification / VIN tag.

21

Tire Safety Information

Reinforced tire

A tire designed to operate at higher loads and at higher inflation pressures than the corresponding standard

tire.

Rim

A metal support for a tire or a tire and tube assembly upon which the tire beads are seated.

Rim diameter

This means the nominal diameter of the bead seat.

Rim size designation

This means the rim diameter and width.

Rim type designation

This means the industry of manufacturer’s designation for a rim by style or code.

Rim width

This means the nominal distance between rim flanges.

Section width

The linear distance between the exteriors of the sidewalls of an inflated tire, excluding elevations due to

labeling, decoration, or protective bands.

Sidewall

That portion of a tire between the tread and bead.

Sidewall separation

The parting of the rubber compound from the cord material in the sidewall.

Special Trailer (ST) tire

The "ST" is an indication the tire is for trailer use only.

Test rim

The rim on which a tire is fitted for testing, and may be any rim listed as appropriate for use with that tire.

Tread

That portion of a tire that comes into contact with the road.

Tread rib

A tread section running circumferentially around a tire.

Tread separation

Pulling away of the tread from the tire carcass.

Treadwear indicators (TWI)

The projections within the principal grooves designed to give a visual indication of the degrees of wear of the

tread.

Vehicle capacity weight

The rated cargo and luggage load plus 68 kilograms (150 lbs.) times the vehicle’s designated seating capacity.

Vehicle maximum load on the tire

The load on an individual tire that is determined by distributing to each axle its share of the maximum loaded

vehicle weight and dividing by two.

22

Tire Safety Information

Vehicle normal load on the tire

The load on an individual tire that is determined by distributing to each axle its share of the curb weight,

accessory weight, and normal occupant weight and dividing by 2.

Weather side

The surface area of the rim not covered by the inflated tire.

Wheel center member

In the case of a non-pneumatic tire assembly incorporating a wheel, a mechanical device which attaches, either

integrally or separably, to the non-pneumatic rim and provides the connection between the non-pneumatic rim

and the vehicle; or, in the case of a non-pneumatic tire assembly not incorporating a wheel, a mechanical

device which attaches, either integrally or separably, to the non-pneumatic tire and provides the connection

between tire and the vehicle.

Wheel-holding fixture

The fixture used to hold the wheel and tire assembly securely during testing.

2.6. TIRE SAFETY - EVERYTHING RIDES ON IT

The National Traffic Safety Administration (NHTSA) has published a brochure (DOT HS 809 361) that

discusses all aspects of Tire Safety, as required by CFR 575.6. This brochure is reproduced in part below. It

can be obtained and downloaded from NHTSA, free of charge, from the following web site:

http://www.nhtsa.dot.gov/cars/rules/TireSafety/ridesonit/tires_index.html

Studies of tire safety show that maintaining proper tire pressure, observing tire and vehicle load limits (not

carrying more weight in your vehicle than your tires or vehicle can safely handle), avoiding road hazards, and

inspecting tires for cuts, slashes, and other irregularities are the most important things you can do to avoid tire

failure, such as tread separation or blowout and flat tires. These actions, along with other care and

maintenance activities, can also:

• Improve vehicle handling

• Help protect you and others from avoidable breakdowns and accidents

• Improve fuel economy

• Increase the life of your tires.

This booklet presents a comprehensive overview of tire safety, including information on the following topics:

• Basic tire maintenance

• Uniform Tire Quality Grading System

• Fundamental characteristics of tires

• Tire safety tips.

Use this information to make tire safety a regular part of your vehicle maintenance routine. Recognize that the

time you spend is minimal compared with the inconvenience and safety consequences of a flat tire or other tire

failure.

2.7. SAFETY FIRST–BASIC TIRE MAINTENANCE

Properly maintained tires improve the steering, stopping, traction, and load-carrying capability of your vehicle.

Underinflated tires and overloaded vehicles are a major cause of tire failure. Therefore, as mentioned above, to

avoid flat tires and other types of tire failure, you should maintain proper tire pressure, observe tire and vehicle

load limits, avoid road hazards, and regularly inspect your tires.

2.8. FINDING YOUR VEHICLE'S RECOMMENDED TIRE PRESSURE &  LOAD LIMITS

Tire information placards and vehicle certification labels contain information on tires and load limits. These

labels indicate the vehicle manufacturer's information including:

• Recommended tire size

23

Tire Safety Information

• Recommended tire inflation pressure

• Vehicle capacity weight (VCW–the maximum occupant and cargo weight a vehicle is designed to carry)

• Front and rear gross axle weight ratings (GAWR– the maximum weight the axle systems are designed to

carry).

Both placards and certification labels are permanently attached to the trailer near the left front.

2.9. UNDERSTANDING TIRE PRESSURE & LOAD LIMITS

Tire inflation pressure is the level of air in the tire that provides it with load-carrying capacity and affects the

overall performance of the vehicle. The tire inflation pressure is a number that indicates the amount of air

pressure– measured in pounds per square inch (psi)–a tire requires to be properly inflated. (You will also find

this number on the vehicle information placard expressed in kilopascals (kPa), which is the metric measure

used internationally.)

Manufacturers of passenger vehicles and light trucks determine this number based on the vehicle's design load

limit, that is, the greatest amount of weight a vehicle can safely carry and the vehicle's tire size. The proper tire

pressure for your vehicle is referred to as the "recommended cold inflation pressure." (As you will read below, it

is difficult to obtain the recommended tire pressure if your tires are not cold.) Because tires are designed to be used on more than one type of vehicle, tire manufacturers list the "maximum permissible inflation pressure" on the tire sidewall. This number is the greatest amount of air pressure that should ever be put in the tire under normal driving conditions.

2.10. CHECKING TIRE PRESSURE

It is important to check your vehicle's tire pressure at least once a month for the following reasons:

• Most tires may naturally lose air over time.

• Tires can lose air suddenly if you drive over a pothole or other object or if you strike the curb when

parking.

• With radial tires, it is usually not possible to determine under inflation by visual inspection.

For convenience, purchase a tire pressure gauge to keep in your vehicle. Gauges can be purchased at tire

dealerships, auto supply stores, and other retail outlets.

The recommended tire inflation pressure that vehicle manufacturers provide reflects the proper psi when a tire

is cold. The term cold does not relate to the outside temperature. Rather, a cold tire is one that has not been

driven on for at least three hours. When you drive, your tires get warmer, causing the air pressure within them

to increase. Therefore, to get an accurate tire pressure reading, you must measure tire pressure when the tires

are cold or compensate for the extra pressure in warm tires.

2.11. STEPS FOR MAINTAINING PROPER TIRE PRESSURE

• Step 1: Locate the recommended tire pressure on the vehicle's tire information placard, certification

label, or in the owner's manual.

• Step 2: Record the tire pressure of all tires.

• Step 3: If the tire pressure is too high in any of the tires, slowly release air by gently pressing on the tire

valve stem with the edge of your tire gauge until you get to the correct pressure.

• Step 4: If the tire pressure is too low, note the difference between the measured tire pressure and the

correct tire pressure. These "missing" pounds of pressure are what you will need to add.

• Step 5: At a service station, add the missing pounds of air pressure to each tire that is underinflated.

• Step 6: Check all the tires to make sure they have the same air pressure (except in cases in which the

front and rear tires are supposed to have different amounts of pressure).

If you have been driving your vehicle and think that a tire is underinflated, fill it to the recommended cold

inflation pressure indicated on your vehicle's tire information placard or certification label. While your tire may

still be slightly underinflated due to the extra pounds of pressure in the warm tire, it is safer to drive with air

pressure that is slightly lower than the vehicle manufacturer's recommended cold inflation pressure than to

drive with a significantly underinflated tire. Since this is a temporary fix, don't forget to recheck and adjust the

tire's pressure when you can obtain a cold reading.

24

Tire Safety Information

2.12. TIRE SIZE

To maintain tire safety, purchase new tires that are the same size as the vehicle's original tires or another size

recommended by the manufacturer. Look at the tire information placard, the owner's manual, or the sidewall of

the tire you are replacing to find this information. If you have any doubt about the correct size to choose,

consult with the tire dealer.

2.13. TIRE TREAD

The tire tread provides the gripping action and traction that prevent your vehicle from slipping or sliding,

especially when the road is wet or icy. In general, tires are not safe and should be replaced when the tread is

worn down to 1/16 of an inch. Tires have built-in tread wear indicators that let you know when it is time to

replace your tires. These indicators are raised sections spaced intermittently in the bottom of the tread grooves.

When they appear "even" with the outside of the tread, it is time to replace your tires. Another method for

checking tread depth is to place a penny in the tread with Lincoln's head upside down and facing you. If you

can see the top of Lincoln's head, you are ready for new tires.

2.14. TIRE BALANCE & WHEEL ALIGNMENT

To avoid vibration or shaking of the vehicle when a tire rotates, the tire must be properly balanced. This

balance is achieved by positioning weights on the wheel to counterbalance heavy spots on the wheel-and-tire

assembly. A wheel alignment adjusts the angles of the wheels so that they are positioned correctly relative to

the vehicle's frame. This adjustment maximizes the life of your tires. These adjustments require special

equipment and should be performed by a qualified technician.

2.15. TIRE REPAIR

The proper repair of a punctured tire requires a plug for the hole and a patch for the area inside the tire that

surrounds the puncture hole. Punctures through the tread can be repaired if they are not too large, but

punctures to the sidewall should not be repaired. Tires must be removed from the rim to be properly inspected

before being plugged and patched.

2.16. TIRE FUNDAMENTALS

Federal law requires tire manufacturers to place standardized information on the sidewall of all tires. This

information identifies and describes the fundamental characteristics of the tire and also provides a tire

identification number for safety standard certification and in case of a recall.

2.17 INFORMATION ON PASSENGER VEHICLE TIRES

Please refer to the diagram below.

25

Tire Safety Information

P

The "P" indicates the tire is for passenger vehicles.

Next number

This three-digit number gives the width in millimeters of the tire from sidewall edge to sidewall edge. In general,

the larger the number, the wider the tire.

Next number

This two-digit number, known as the aspect ratio, gives the tire's ratio of height to width. Numbers of 70 or

lower indicate a short sidewall for improved steering response and better overall handling on dry pavement.

R

The "R" stands for radial. Radial ply construction of tires has been the industry standard for the past 20 years.

Next number

This two-digit number is the wheel or rim diameter in inches. If you change your wheel size, you will have to

purchase new tires to match the new wheel diameter.

Next number

This two- or three-digit number is the tire's load index. It is a measurement of how much weight each tire can

support. You may find this information in your owner's manual. If not, contact a local tire dealer. Note: You may

not find this information on all tires because it is not required by law.

M+S

The "M+S" or "M/S" indicates that the tire has some mud and snow capability. Most radial tires have these

markings; hence, they have some mud and snow capability.

Speed Rating

The speed rating denotes the speed at which a tire is designed to be driven for extended periods of time. The

ratings range from 99 miles per hour (mph) to 186 mph. These ratings are listed below. Note: You may not find

this information on all tires because it is not required by law.

26

Tire Safety Information

| Letter Rating - Speed Rating |  |
| --- | --- |
| Q | 99 mph |
| R | 106 mph |
| S | 112 mph |
| T | 118 mph |
| U | 124 mph |
| H | 130 mph |
| V | 149 mph |
| W | 168* mph |
| Y | 186* mph |

* For tires with a maximum speed capability over 149 mph, tire manufacturers sometimes use the letters ZR.

For those with a maximum speed capability over 186 mph, tire manufacturers always use the letters ZR.

U.S. DOT Tire Identification Number

This begins with the letters "DOT" and indicates that the tire meets all federal standards. The next two numbers

or letters are the plant code where it was manufactured, and the last four numbers represent the week and year

the tire was built. For example, the numbers 3197 means the 31st week of 1997. The other numbers are

marketing codes used at the manufacturer's discretion. This information is used to contact consumers if a tire

defect requires a recall.

Tire Ply Composition and Materials Used

The number of plies indicates the number of layers of rubber-coated fabric in the tire. In general, the greater the

number of plies, the more weight a tire can support. Tire manufacturers also must indicate the materials in the

tire, which include steel, nylon, polyester, and others.

Maximum Load Rating

This number indicates the maximum load in kilograms and pounds that can be carried by the tire.

Maximum Permissible Inflation Pressure

This number is the greatest amount of air pressure that should ever be put in the tire under normal driving

conditions.

2.18. UTQGS INFORMATION

Treadwear Number

This number indicates the tire's wear rate. The higher the tread wear number is, the longer it should take for the

tread to wear down. For example, a tire graded 400 should last twice as long as a tire graded 200.

Traction Letter

This letter indicates a tire's ability to stop on wet pavement. A higher graded tire should allow you to stop your

car on wet roads in a shorter distance than a tire with a lower grade. Traction is graded from highest to lowest

as "AA","A", "B", and "C".

Temperature Letter

This letter indicates a tire's resistance to heat. The temperature grade is for a tire that is inflated properly and

not overloaded. Excessive speed, under inflation or excessive loading, either separately or in combination, can

cause heat build-up and possible tire failure. From highest to lowest, a tire's resistance to heat is graded as "A",

"B", or "C".

27

Tire Safety Information

2.19. ADDITIONAL INFORMATION ON LIGHT TRUCK TIRES

Please refer to the following diagram.

Tires for light trucks have other markings besides those found on the sidewalls of passenger tires.

LT

The "LT" indicates the tire is for light trucks or trailers.

ST

An "ST" is an indication the tire is for trailer use only.

Max. Load Dual kg (lbs) at kPa (psi) Cold

This information indicates the maximum load and tire pressure when the tire is used as a dual, that is, when

four tires are put on each rear axle (a total of six or more tires on the vehicle).

Max. Load Single kg (lbs) at kPa (psi) Cold

This information indicates the maximum load and tire pressure when the tire is used as a single.

Load Range

This information identifies the tire's load-carrying capabilities and its inflation limits.

2.20. TIRE SAFETY TIPS

Preventing Tire Damage

• Slow down if you have to go over a pothole or other object in the road.

• Do not run over curbs or other foreign objects in the roadway, and try not to strike the curb when parking.

Tire Safety Checklist

• Check tire pressure regularly (at least once a month), including the spare.

• Inspect tires for uneven wear patterns on the tread, cracks, foreign objects, or other signs of wear or

trauma.

• Remove bits of glass and foreign objects wedged in the tread.

• Make sure your tire valves have valve caps.

• Check tire pressure before going on a long trip.

• Do not overload your vehicle. Check the Tire Information and Loading Placard or User’s Manual for the

maximum recommended load for the vehicle.

28

Coupling to the Tow Vehicle

3. COUPLING TO THE TOW VEHICLE

Follow all of the safety precautions and instructions in this manual to ensure safety of persons, cargo, and

satisfactory life of the trailer.

3.1 USE AN ADEQUATE TOW VEHICLE & HITCH

If the vehicle or hitch is not properly selected and matched to the Gross Vehicle Weight Rating (GVWR) of your

trailer, you can cause an accident that could lead to death or serious injury. If you already have a tow vehicle,

know your vehicle tow rating, and Gross Combination Weight Rating (GCWR) and make certain the trailer’s

rated capacity is less than or equal to the tow vehicle’s rated towing capacity. If you already have (or plan to

buy) a trailer, make certain that the tow rating of the tow vehicle is equal to or greater than the GVWR of the

trailer, and that the GCWR will be within limits.

^ DANGER

Use of a hitch with a load rating less than the load rating of the trailer can result in loss of control and may lead to death or serious injury. Use of a tow vehicle with a towing capacity less than the load rating of the trailer can result in loss of control, and may lead to death or serious injury. Be sure your hitch and tow vehicle are rated for the Gross Vehicle Weight Rating of your trailer.

29

Coupling to the Tow Vehicle

3.2. TRAILER INFORMATION

Certification / VIN Label

The location of the trailers “Certification / VIN

Tag” is shown below.

The trailer Certification / VIN tag contains the following critical safety information for the use of

your trailer:

• MANUFACTURER: Name of trailer manufacturer

• DATE OF MANUFACTURE: Month and year the trailer was manufactured.

• GVWR: The Gross Vehicle Weight Rating is the maximum allowable gross weight of the

trailer and its contents. The gross weight of the trailer includes the weight of the trailer and all

of the items within it (such as cargo, water, food and other supplies).

30

Coupling to the Tow Vehicle

• GAWR: The Gross Axle Weight Rating is the maximum gross weight that an axle can

support. It is the lowest of axle, wheel, or tire rating. Sometimes the tire or wheel rating is

lower than the axle manufacturers rating, and will then determine GAWR.

The sum total of the GAWR for all trailer axles may be less than the GVWR for the trailer,

because some of the trailer load is carried by the tow vehicle, rather than by the trailer axle(s).

The total weight of the cargo and trailer must not exceed the GVWR, and the load on an axle

must not exceed its GAWR.

• TIRE SIZE: The tire size recommended for your trailer and load range.

• PSIC: The “pounds per square inch- cold” is the tire pressure (Kilopascals / Pounds per

Square Inch) measured when Cold.

• CERTIFICATION STATEMENT: “This trailer meets all the Federal Motor Vehicle Safety

Standards in effect on the date of manufacture shown above”.

• VIN: The Vehicle Identification Number.

• VEHICLE TYPE: Generally the word “trailer” is used. However, after this you may put a

Model #, or additional descriptor.

• Tow Vehicle: When equipping a new vehicle or an older vehicle to tow your trailer, ask the

vehicle dealer for advice on how to outfit the towing vehicle. Discuss the following information

and equipment with the vehicle dealer.

• Overall Carrying and Towing Capacity of Vehicle: Vehicle manufacturers will provide you

with the maximum towing capacities of their various models, as well as the GCWR. No

amount of reinforcement will give a 100 horsepower, 2,500 pound truck the towing capacity

that a 300 horsepower, 5,000 pound truck has.

• Towing Hitch: The towing hitch attached to your tow vehicle must have a capacity equal to or

greater than the load rating of the trailer you intend to tow. The hitch capacity must also be

matched to the tow vehicle capacity.

• Suspension System: A tow vehicle equipped with a factory installed “Towing Package” likely

comes equipped with heavy duty springs, heavy duty tires and other suspension components

which are able to serve the size and weight of the trailer that the vehicle is rated to tow.

However, the addition of additional equipment may further improve the tow vehicle

performance. These may include adjustable air shocks, helper springs, etc.

• Brake Controller: The brake controller is part of the tow vehicle and is essential in the

operation of the electric brakes on the trailer. If your trailer has electric brakes it requires a

brake controller be installed at the driver’s position. The brake controller is not the same as

the safety breakaway brake system that is installed on the trailer.

• Side View Mirrors: The size of the trailer that is being towed and your state law regulations

determine the size of the mirrors. However, some states prohibit extended mirrors on a tow

vehicle, except while a trailer is actually being towed. In this situation, detachable extended

mirrors are necessary. Check with your dealer or the appropriate state agency for mirror

requirements.

• Heavy Duty Flasher: A Heavy Duty Flasher is an electrical component that may be required

when your trailer turn signal lights are attached to the tow vehicle flasher circuit.

31

Coupling to the Tow Vehicle

• Electrical Connector: An Electrical Connector connects the light and brake systems on the

trailer to the light and brake controls on the towing vehicle.

• Heavy Duty Engine Oil Cooling System: The tow vehicle engine works harder when a trailer

is being towed. Depending on the size of the trailer, you may need to install a separate

engine oil cooler. Inadequate cooling may result in sudden engine failure. Ask the tow

vehicle dealer if it is necessary to install a heavy duty cooling system.

• Automatic Transmission Oil Cooler: The automatic transmission of a towing vehicle

handles more power when a trailer is being towed. Inadequate cooling will shorten

transmission life, and may result in sudden transmission failure. Ask the tow vehicle dealer if

it is necessary to install a separate oil cooler for the automatic transmission.

• Fire Extinguisher: It is sensible to have a fire extinguisher in the tow vehicle.

• Emergency Flares and Emergency Triangle Reflectors: It is wise to carry these warning

devices even if you are not towing a trailer. It is particularly important to have these when

towing a trailer because the hazard flashers of your towing vehicle will not operate for as long

a period of time when the battery is running both the trailer lights and tow vehicle lights.

3.3 COUPLING & UNCOUPLING THE TRAILER

A secure coupling (or fastening) of the trailer to the tow vehicle is essential. A loss of coupling

may result in death or serious injury. Therefore, you must understand and follow all of the

instructions for coupling.

The following parts are involved in making a secure coupling between the trailer and tow vehicle:

Coupling: That part of the trailer connecting mechanism by which the connection is actually

made to the trailer hitch. This does not include any structural member, extension of the trailer

frame, or brake controller.

Hitch: That part of the connecting mechanism including the ball support platform and ball and

those components that extend and are attached to the towing vehicle, including bumpers intended

to serve as hitches.)

• Weight Distributing Hitch (or Equalizing Hitch): A mechanical device that connects

the trailer to the towing vehicle and by means of leverage applied on both the trailer and

towing vehicle structures, when properly adjusted, distributes the imposed vertical load at

the hitch and coupling connection between structures of the towing vehicle and trailer.

• Weight Carrying Hitch: A mechanical and/or structural device that connects the trailer to

the towing vehicle and that does not employ features designed to redistribute the load

imposed at the hitch and carrying connection.

Safety chains or cables: Chains or cables permanently attached to the trailer such that if the

coupler connection comes loose, the safety chains or cables can keep the trailer attached to the

tow vehicle. With properly rigged safety chains or cables, it is possible to keep the tongue of the

trailer from digging into the road pavement, even if the coupler-to-hitch connection comes apart.

Some states do not allow safety cables, e.g. Pennsylvania; therefore it may be wise to check with

the State Police to see if your state has any restrictions on the use of safety cables, if your trailer

is so equipped.

Trailer lighting (and braking) connector: A device that connects electrical power from the tow

vehicle to the trailer. Electricity is used to turn on brake lights, running lights, and turn signals as

32

Coupling to the Tow Vehicle

required. In addition, if your trailer has a separate braking system, the electrical connector will

also supply power to the trailer brakes from the tow vehicle.

Breakaway switch: If the trailer becomes de-coupled from the towing vehicle, the breakaway

switch lanyard, attached independently to the tow vehicle hitch, will pull a pin in the emergency

electrical break-away switch on the trailer. The breakaway switch is activated by a separate

battery supply in the trailer such as to energize the trailer brakes independently of the towing

vehicle. It is important to check the state of charge of the emergency break-away battery before

each trip. Simply pull the pin out of the switch by hand and then try to pull the trailer. If you feel a

significant drag force the brakes are activated. Be sure to re-insert the pin in the break-away

switch. Also be sure to allow enough slack in the break-away brake lanyard such that the switch

will only activate (pin pulls out) if the coupler connection comes loose.

Jack: A device on the trailer that is used to raise and lower the trailer tongue. On larger trailers

the jack is sometimes called the “landing gear.”

^

^WARNING

An improperly coupled trailer can result in death or serious injury. Do not move the trailer until: • The coupler is secured and locked to hitch; • The safety chains are secured to the tow vehicle; and • The trailer jack(s) are fully retracted. Do not tow the trailer on the road until: • Tires and wheels are checked; • The trailer brakes are checked; • The breakaway switch is connected to the tow vehicle, and the break-away battery is checked. • The load is secured to the trailer; and • The trailer lights are connected and checked.

3.4. VARIOUS COUPLER DESIGNS

Trailers are produced with a variety of coupler devices. One of the sections below will pertain to

your trailer.

Bumper pull (Ball Hitch) Coupler

Gooseneck Hitch Coupler

If the coupler on your trailer does not resemble one of the couplers shown in the figures, see the

separate coupler instructions. If you do not have separate coupler instructions, call Thuro-Bilt, Inc. at 541-670-8701

for a free copy.

33

Coupling to the Tow Vehicle

3.5.  TRAILER WITH BALL-HITCH COUPLER

A ball hitch coupler connects to a ball that is located on or under the rear bumper of tow vehicle.

This system of coupling a trailer to a tow vehicle is sometimes referred to as “bumper pull.”

A ball hitch trailer may be fitted with a tongue jack that can raise and lower the coupler. The

tongue jack is mounted to the A-frame (front, or tongue) part of the trailer. By rotating the jack

handle clockwise, the jack will extend and raise the tongue of the trailer.

Trailer with Ball-hitch Coupler Pictured Below

Be sure the Ball Hitch coupler is suitable for the size and weight of the trailer. The load rating of

the coupler and the necessary ball size are listed on the trailer tongue. You must provide a hitch

and ball for your tow vehicle, where the load rating of the hitch and ball is equal to or greater than

that of your trailer. Also, the ball size must be the same as the coupler size. If the hitch ball is too

small, too large, is underrated, is loose or is worn, the trailer can come loose from the tow vehicle,

and may cause death or serious injury.

THE TOW VEHICLE, HITCH AND BALL MUST HAVE A RATED TOWING CAPACITY EQUAL

TO OR GREATER THAN THE TRAILER GROSS VEHICLE WEIGHT RATING (GVWR).

IT IS ESSENTIAL THAT THE HITCH BALL BE OF THE SAME SIZE AS THE COUPLER.

The ball size and load rating (capacity) are marked on the ball; hitch capacity is marked on the

hitch.

3.6.  BEFORE COUPLING THE TRAILER TO THE TOW VEHICLE

Be sure the size and rating of hitch ball match the size and rating of the coupler. Hitch balls and

couplers are marked with their size and rating.

34

Coupling to the Tow Vehicle

^

^WARNING

Coupler-to-hitch mismatch can result in uncoupling, leading to death or serious injury. Be sure the LOAD RATING of the hitch ball is equal or greater than the load rating of the coupler. Be sure the SIZE of the hitch ball matches the size of the coupler.

Wipe the hitch ball clean and inspect it visually and by feel for flat spots, cracks and pits.

^WARNING

A worn, cracked or corroded hitch ball can fail while towing, and may result in death or serious injury. Before coupling trailer, inspect the hitch ball for wear, corrosion and cracks. Replace worn or damaged hitch ball.

Rock the ball to make sure it is tight to the hitch, and visually check that the hitch ball nut is

solid against the lock washer and hitch frame.

Wipe the inside and outside of the coupler clean and inspect it visually for cracks and

deformations; feel the inside of the coupler for worn spots and pits.

Be sure the coupler is tight to the tongue of the trailer. All coupler fasteners must be visibly

solid against the trailer frame.

^WARNING

A loose hitch-ball nut can result in uncoupling, leading to death or serious injury. Be sure the hitch ball is tight to the hitch before coupling the trailer.

Raise the bottom surface of the coupler to be above the top of the hitch ball. Use the jack if

one is provided; otherwise, use wood or concrete blocks to support the trailer tongue.

3.7. PREPARE THE COUPLER & HITCH

Lubricate the hitch ball and the inside of the coupler with a thin layer of automotive bearing

grease. If your trailer is equipped with a jack, raise the coupler above the ball height.

35

Coupling to the Tow Vehicle

Open the coupler locking mechanism. Ball couplers have a locking mechanism with an

internal moving piece (ball clamp) and an outside handle, wheel, or latch.

In the open position, the coupler is able to drop fully onto the hitch ball.

See the coupler instructions for details of placing the coupler in the “open” position.

Slowly back up the tow vehicle so that the hitch ball is near or aligned under the coupler, if the

trailer jack has raised the coupler.

Figure 3-1 Ball Hitch Coupler Engaged

Closed

Released

3.8. COUPLE THE TRAILER TO THE TOW VEHICLE

If your trailer does not have a jack, you will have to lift the coupler and place it over the ball.

If you have a jack, lower the trailer tongue until the coupler fully engages the hitch ball. If the

coupler does not line up with the hitch ball, adjust the position of the tow vehicle.

Engage the coupler locking mechanism. In the engaged position, the locking mechanism

securely holds the coupler to the hitch ball.

Insert a pin or lock through the hole in the locking mechanism.

36

Coupling to the Tow Vehicle

Be sure the coupler is all the way on the hitch ball and the locking mechanism is engaged. A

properly engaged locking mechanism will allow the coupler to raise the rear of the tow

vehicle. Using the trailer jack, test to see that you can raise the rear of the tow vehicle by

1 inch, after the coupler is locked to the hitch

Safety Chain Arrangement

^NOTICE

Overloading can damage the tongue jack. Do not use the tongue jack to raise the tow vehicle more than 1 inch.

If the coupler cannot be secured to the hitch ball, do not tow the trailer. Call Thuro-Bilt, Inc. at 541-670-8701

or your dealer for assistance.

Lower the trailer so that its entire tongue weight is held by the hitch, and continue retracting

the jack to its fully retraced position.

3.9. RIG THE SAFETY CHAINS

Visually inspect the safety chains and hooks for wear or damage. Replace worn or damaged

safety chains and hooks before towing.

Rig the safety chains so that they:

Criss-cross underneath the coupler so if the trailer uncouples, the safety chains can hold

the tongue up above the road.

37

Coupling to the Tow Vehicle

Loop around a frame member of the tow vehicle or to holes provided in the hitch system

(but, do not attach them to an interchangeable part of the hitch assembly)

Attach “S” hooks up from underneath the hole (do not just drop into hole); and

Provide enough slack to permit tight turns, but not be close to the road surface to drag.

^

^WARNING

Improper rigging of the safety chains can result in loss of control of the trailer and tow vehicle, leading to death or serious injury, if the trailer uncouples from the tow vehicle. • Fasten chains to frame of tow vehicle. Do not fasten chains to any part of the hitch unless the hitch has holes or loops specifically for that purpose. • Cross chains underneath hitch and coupler with enough slack to permit turning and to hold tongue up, if the trailer comes loose.

3.10. ATTACH & TEST ELECTRIC BREAKWAY SYSTEM

If the coupler or hitch fails, a properly connected and working breakaway brake system will apply

electric brakes on the trailer. The safety chains will keep the tow vehicle attached and as the

brakes are applied at the trailer’s axles, the trailer/tow vehicle combination will come to a

controlled stop.

The breakaway brake system includes a battery, a switch with a pullpin, and a lanyard. Read and

follow the instructions here as well as the instructions that have been prepared by the breakaway

brake manufacturer. If you do not have these instructions, call Thuro-Bilt, Inc. at 541-670-8701

for a free copy.

The breakaway brake system may be fitted with a “charging” capability that draws power from the

tow vehicle

38

Coupling to the Tow Vehicle

Break away Switch and Battery options Bumper Pull Trailer

Connect the pullpin lanyard to the tow vehicle so that the pullpin will be pulled out before all of

the slack in the safety chains is taken up (see Breakaway Brake System figure). Do not

connect the pullpin cable to a safety chain or to the hitch ball or hitch ball assembly. This

would keep the breakaway brake system from operating when it is needed.

To test the break-away brake battery, remove the pullpin from the switch and attempt to pull

the trailer forward. You should feel the trailer resisting being towed, but the wheels will

not necessarily be locked. If the brakes do not function, do not tow the trailer until brakes,

or battery, are repaired.

Immediately replace the pullpin. The breakaway brake system battery discharges rapidly

when the pullpin is removed.

^

^WARNING

An ineffective breakaway brake system can result in a runaway trailer, leading to death or serious injury, if the coupler or ball hitch fails. Connect the breakaway cable to the tow vehicle; and NOT to the hitch, ball or support. Before towing the trailer, test the function of the breakaway brake system. If the breakaway brake system is not working, do not tow the trailer. Have it serviced or repaired.

Do not tow the trailer with the breakaway brake system ON because the brakes will overheat

which can result in permanent brake failure.

39

Coupling to the Tow Vehicle

^WAR

^WARNING

Failure to replace the pullpin will prevent brakes from working, leading to loss of control, serious injury or death.

If you do not use your trailer for three or more months, or during winter months:

• Store the battery indoors; and

• Charge the battery every three months.

Replace the breakaway brake battery according to the intervals specified by battery manufacturer.

40

Coupling to the Tow Vehicle

3.11. CONNECT THE ELECTRICAL CABLES

Connect the trailer lights to the tow vehicle's electrical system using the electrical connectors.

Check all lights for proper operation.

Clearance and Running Lights (Turn on tow vehicle headlights).

Brake Lights (Step on tow vehicle brake pedal).

Turn Signals (Operate tow vehicle directional signal lever).

Check electric brakes for proper operation using brake controller mounted in the cab.

If your trailer has electric brakes, your tow vehicle will have an electric brake controller that sends

power to the trailer brakes. Before towing the trailer on the road, you must operate the brake

controller while trying to pull the trailer in order to confirm that the electric brakes operate. While

towing the trailer at less than 5 m.p.h., manually operate the electric brake controller in the tow

vehicle cab. You should feel the operation of the trailer brakes.

^W

^WARNING

Improper electrical connection between the tow vehicle and the trailer will result in inoperable lights and electric brakes, and can lead to collision. Before each tow: • Check that the taillights, brake lights and turn signals work • Check that the electric brakes work by operating the brake controller inside the tow vehicle.

3.12. UNCOUPLING THE BALL HITCH TRAILER WITH TONGUE JACK

Follow these steps to uncouple your ball hitch trailer from the tow vehicle:

• Block trailer tires to prevent the trailer from rolling, before jacking the trailer up.

• Disconnect the electrical connector.

• Disconnect the breakaway brake switch lanyard.

• Disconnect the safety chains from the tow vehicle.

• Unlock the coupler and open it.

• Before extending jack, make certain the ground surface below the jack pad will

support the tongue load.

• Rotate the jack handle (or crank) clockwise. This will slowly extend the jack and

transfer the weight of the trailer tongue to the jack.

3.13. TRAILER WITH GOOSENECK COUPLER & DROP-LEG JACK

A gooseneck coupler on the trailer connects to a gooseneck ball that you must have installed in

the bed of the tow vehicle. This system of coupling a trailer to a tow vehicle permits the tow

vehicle to turn to sharper angles than are permitted by a bumper hitch system. A gooseneck

coupler consists of a tube in an inverted “U” shape and a gooseneck ball receiver. “Trailer with

Gooseneck Hitch Coupler” figure shows a trailer with a gooseneck coupler.

41

Coupling to the Tow Vehicle

Trailer with Gooseneck Coupler

We have installed a Gooseneck ball receiver that is suitable for the size and weight of the trailer.

The load rating of the coupler and the necessary ball size are listed on the gooseneck.

You must provide a gooseneck ball and support structure that is marked with a rating that meets

or exceeds the GVWR of your trailer and matches the size of the gooseneck ball receiver. If the

gooseneck ball is too small, is underrated, is loose or is worn, the trailer can come loose from the

tow vehicle, and may lead to death or serious injury.

THE TOW VEHICLE, SUPPORT STRUCTURE AND GOOSENECK BALL MUST HAVE A

RATED TOWING CAPACITY EQUAL TO OR GREATER THAN THE TRAILER GROSS

VEHICLE WEIGHT RATING (GVWR).

IT IS ESSENTIAL THAT THE GOOSENECK BALL BE OF THE SAME SIZE AS THE

GOOSENECK BALL RECEIVER.

The ball size and load rating (capacity) are marked on the ball; hitch capacity is marked on the

hitch.

^W

^WARNING

Coupler-to-hitch mismatch can result in uncoupling, leading to death or serious injury. Be sure the LOAD RATING of the hitch ball is equal or greater than the load rating of thecoupler. Be sure the SIZE of the hitch ball matches the size of the coupler.

42

Coupling to the Tow Vehicle

The height of the ball receiver on the trailer must be adjusted to match the height of the

gooseneck ball on your tow vehicle, so that:

There is clearance between the bottom of the trailer and the sides of the tow vehicle bed;

and

The trailer is level and allows equal weight distribution on tandem axles.

The “Gooseneck Ball Receiver and Height Adjustment” figure shows the gooseneck height

adjustment. The gooseneck height adjustment bolts, which have a “cup” that makes a gripping

impression into the gooseneck tube, must be tight so that the trailer does not drop to a lower

position. Do not over-tighten because the tube can be deformed. After tightening the bolts,

tighten the jam nuts on the bolts.

Gooseneck Ball Receiver and Height Adjustment

^WARNING

Improper gooseneck height adjustment can result in overloaded tires, blowout and loss of control, leading to death or serious injury. Adjust the gooseneck receiver so that the loaded trailer is level.

A trailer having a gooseneck hitch will have one or two drop leg jacks for raising and lowering the

gooseneck ball receiver. Because several drop leg jack mechanisms are available, the general

instructions below may vary slightly from the jack manufacturer’s instructions. If the trailer jack on

your trailer does not resemble the jack shown in the figures, follow the jack instructions provided

by the jack manufacturer. If you do not have these instructions, call Thuro-Bilt, Inc. at

541-670-8701 for a free copy.

43

Coupling to the Tow Vehicle

Drop-leg Jack Arrangement

Before attempting to tow the trailer:

Be sure the size and rating of the gooseneck ball match the size and rating of the receiver.

Gooseneck balls and receivers are marked with their size and ratings.

Wipe the gooseneck ball clean and inspect it visually and by feel for flat spots, cracks and

pits.

44

Coupling to the Tow Vehicle

^

^WARNING

A worn, cracked or corroded gooseneck ball can fail while towing, and may result in death or serious injury. Before coupling the trailer, inspect the gooseneck ball for wear, corrosion and cracks; and replace worn or damaged gooseneck ball.

Rock the ball to make sure it is tight to the ball support, and visually check that the gooseneck

ball nut is solid against the lock washer and ball support frame.

^

^WARNING

A loose gooseneck ball can result in uncoupling, leading to death or serious injury. Be sure the gooseneck ball nut is tight before coupling the trailer.

Wipe the inside and outside of the receiver clean and inspect it visually for cracks; and feel

the inside of the receiver for worn spots and pits. If any of these conditions exist, have the

receiver replaced before coupling the trailer.

Lubricate the inside of the gooseneck ball receiver with automotive bearing grease.

Be sure the receiver is tight to the trailer. All receiver fasteners must be visibly solid against

the trailer frame.

Release the jack handle or crank from its holder (see “Drop Leg Jack” figure).

Make certain the ground beneath the jack foot is firm enough to support the tongue weight.

Rotate the handle/crank clockwise to raise the bottom surface of the gooseneck to be above

the top of the gooseneck ball.

3.14. PREPARE THE BALL RECEIVER & GOOSENECK BALL

Release the lock plate on the gooseneck ball receiver. With the spring-loaded lock plate

locking pin in the OPEN position, rotate the lock plate to a position that allows the

gooseneck ball to enter the receiver (see “Gooseneck Ball Receiver and Height

Adjustment” figure).

Slowly back up the tow vehicle so that the gooseneck ball is aligned under the gooseneck ball

receiver.

45

Coupling to the Tow Vehicle

^W^

^WARNING

If the trailer drops during coupling, death or serious injury may result. There must be no one under the trailer or coupler before or during the coupling operation.

3.15. COUPLE THE TRAILER TO THE TOW VEHICLE

Rotate the jack handle counter-clockwise. This will retract the jack causing the gooseneck

ball receiver to drop down so it can fully engage the gooseneck ball and transfer the

weight of the trailer tongue to the towing vehicle hitch. If the receiver does not line up with

the ball, raise the receiver again and adjust the position of the tow vehicle. Then lower

the receiver over the ball. When the drop leg base is no longer resting on the ground, the

towing vehicle hitch is holding all of the weight of the trailer tongue.

Close the lock plate on the gooseneck ball receiver.

Move the spring-loaded lock plate locking pin to the CLOSED position. Be sure the locking

pin is holding the lock plate.

Be sure the receiver is all the way on the gooseneck ball and the lock plate is engaged. A

properly engaged locking mechanism will allow the coupler to raise the rear of the tow

vehicle. Using the trailer jack, test to see that you can raise the rear of the tow vehicle by

1 inch.

^NOTICE

Overloading can damage the drop leg jack. Do not use the drop leg jack to raise the tow vehicle more than 1 inch.

If the gooseneck ball cannot be secured to the receiver, do not tow the trailer. Call

Thuro-Bilt, Inc. at 541-670-8701 or your dealer for assistance.

After testing to see that the receiver is properly secured and locked to the ball, retract the jack

to its fully retracted position.

Return the drop legs to their upper positions. The drop legs are held in the lowered position

with a plunger pin. Rotating the plunger pin while pulling it outward will cause it to come

out of engagement with the drop leg and the leg will rapidly rise (see “Drop Leg

Mechanism” figure).

46

Coupling to the Tow Vehicle

Drop Leg Mechanism

^ CAUTION

The drop legs are heavily spring loaded in the Lowered position. They will rapidly return to the upper position when released and can inflict serious bruises, scrapes or pinching. Keep your feet, shins and hands well clear of the drop legs and drop leg bases when releasing the drop legs. Always wear shoes or boots while performing this operation.

3.16. RIG THE SAFETY CHAINS

Visually inspect the safety chains and hooks for wear or damage. Replace worn or damaged

safety chains and hooks before towing.

Rig the safety chains so that they attach to the “safety chain receivers” in the bed of the

truck. If you are not certain of the hitch provisions for receiving safety chains, contact the

hitch manufacturer or installer. Do NOT attach the safety chains to the gooseneck ball or

its support; and

47

Coupling to the Tow Vehicle

Rig the safety chains so they have sufficient slack to permit turning, but not too much

slack – the safety chains must keep the gooseneck on the tow vehicle bed if the trailer

uncouples.

^WARNING

Improper rigging of the safety chains can result in loss of control of the trailer and tow vehicle, leading to death or serious injury, if the trailer uncouples from the tow vehicle. • Fasten chains to safety chain receivers on the hitch, not to ball. • Have sufficient slack to permit turning and to keep gooseneck on bed of tow vehicle, if the trailer comes loose.

3.17. ATTACH & TEST THE BREAKAWAY BRAKE SYSTEM

If the coupler or hitch fails, a properly connected and working breakaway brake system will apply

electric brakes on the trailer. The safety chains will keep the tow vehicle attached and as the

brakes are applied at the trailer’s axles, the trailer/tow vehicle combination will come to a

controlled stop.

The breakaway brake system includes a battery, a switch with a pullpin, and a breakaway brake

controller. Read and follow the instructions here as well as the instructions that have been

prepared by the breakaway brake controller manufacturer. If you do not have these instructions,

call Thuro-Bilt, Inc. at 541-670-8701 for a free copy.

Breakaway Brake System Gooseneck Coupler

The breakaway brake system may be fitted with a charging facility that draws power from the tow

vehicle. If the electrical system on your tow vehicle does not provide power to the breakaway

48

Coupling to the Tow Vehicle

brake battery you must periodically charge the battery on the trailer to keep the breakaway brake

system in working order.

Visually inspect the breakaway brake system for broken parts.

Connect the pullpin cable to the tow vehicle so that the pullpin will be pulled out before all of

the slack in the safety chains is taken up (see “Safety Chains” figure). Do not connect the

pullpin cable to a safety chain or a safety chain receiver or to the gooseneck ball or its

support. This would keep the breakaway brake system from operating when it is needed.

Contact the hitch manufacturer or installer if you are not certain of the hitch provisions for

breakaway brake connection

To check the break-away brake battery pull out the pullpin from the switch and attempt to pull

the trailer forward. You should feel the trailer resisting being towed, but the wheels will not

necessarily lock up.

Immediately replace the pullpin. The breakaway brake system battery discharges rapidly

when the pullpin is removed.

^WARNING

An ineffective or inoperative breakaway brake system can result in a runaway trailer leading to death or serious injury, if the coupler or hitch fails. Connect the breakaway cable to the tow vehicle; and NOT to the safety chain, safety chain receiver, gooseneck ball or gooseneck ball support. Test the function of the breakaway brake system before towing the trailer. Do not tow the trailer if the breakaway brake system is not working; have it serviced or repaired.

Do not tow the trailer with the breakaway brake system ON because the brakes will overheat

which can result in permanent brake failure.

^

^WARNING

Failure to replace the pullpin will prevent brakes from working, leading to loss of control, serious injury or death.

If you do not use your trailer for three or more months, or during winter months:

Store the battery indoors; and

Charge the battery every three months.

Replace the breakaway brake battery at intervals recommended by the battery manufacturers

instructions.

49

Coupling to the Tow Vehicle

3.18. CONNECT THE ELECTRICAL CABLES

Connect the trailer lights to the tow vehicle's electrical system using the electrical connectors.

Check all lights for proper operation:

Clearance and Running Lights (Turn on tow vehicle headlights).

Brake Lights (Step on tow vehicle brake pedal).

Turn Signals (Operate tow vehicle directional signal lever).

Check electric brakes for proper operation

If your trailer has electric brakes, your tow vehicle will have an electric brake controller that sends

power to the trailer brakes. Before towing the trailer on the road, you must operate the brake

controller while trying to pull the trailer in order to confirm that the electric brakes operate. While

towing the trailer at less than 5 m.p.h., manually operate the electric brake controller in the tow

vehicle cab. You should feel the operation of the trailer brakes.

^WARNING

Improper electrical connection between the tow vehicle and the trailer will result in inoperable lights and electric brakes, and can lead to collision. Before each tow: • Check that the taillights, brake lights and turn signals work. • Check that the electric brakes work by operating the brake controller inside the tow vehicle.

3.19. UNCOUPLING THE GOOSENECK TRAILER WITH DROP-LEG JACK

Follow these steps to uncouple your gooseneck hitch trailer from the tow vehicle:

• Block trailer tires to prevent the trailer from rolling, before jacking the trailer up

• Disconnect the electrical connector.

• Disconnect the breakaway brake switch lanyard.

• Disconnect the safety chains from the tow vehicle.

• Move the spring-loaded gooseneck receiver lock plate locking pin to the OPEN

position (see “Gooseneck Ball Receiver and Height Adjustment” figure).

• Rotate the lock plate to a position that permits the gooseneck ball to exit the receiver.

• Before releasing dropleg jack, make certain ground surface below jack base will

support the trailer tongue load.

• Rotate the drop leg plunger pin handle so that the plunger pin is released from the

drop leg (see “Releasing Drop Leg Mechanism” figure).

• Push down on the drop leg base with your foot to place a drop leg to the desired

lowered position.

• Rotate the plunger pin handle so that the plunger pin is attempting to engage the drop

leg (see “Releasing Drop Leg Mechanism” figure)

50

Coupling to the Tow Vehicle

• Slowly raise your foot, permitting the drop leg to raise. The plunger pin will engage a

hole in the drop leg.

^ CAUTION

The drop legs are heavily spring loaded in the lowered position. They will rapidly return to the upper position when released and can inflict serious bruises, scrapes or pinching. Keep your feet, shins and hands well clear of the drop legs and drop leg bases when releasing the drop legs. Always wear shoes or boots while performing this operation.

• Be sure the plunger pin is fully engaged. Push it in by hand if necessary. The bent

part of the plunger pin handle must be touching the plunger pin housing.

• If your trailer has two drop leg jacks, lower them both to the same level, following the

above instructions.

^NOTICE

If the drop legs are not set at the same level, one of the drop leg jacks can be overloaded and can be damaged.

• Release the handle (or crank) from its holder and engage it with the jack shaft (see

“Drop Leg Jack” figure).

• Rotate the handle (or crank) from its hold and engage it with the jack shaft (see “Drop

Leg Jack” figure).

• Rotate the handle (or crank) clockwise to slowly extend the jack and transfer the

weight of the trailer tongue to the jack.

• On two speed jacks, pushing the handle shaft toward the gearbox can perform rapid

extension. This shifts the gearbox into a high speed mode.

• When the drop leg base contacts the ground, shift the gearbox into low gear mode by

pulling or pushing on the handle shaft until it locks into low gear.

^NOTICE

Do not use high gear to lift the trailer; the drop leg jack mechanism can be damaged. High gear is used only to rapidly move the drop leg base into contact with the ground.

51

Coupling to the Tow Vehicle

• Continue to extend the jack(s), making sure that the ground is providing stable and level

support for the trailer.

• After the jack(s) are extended and the gooseneck ball receiver is well clear of the

gooseneck ball, to permit driving the tow vehicle away, disengage the handle from its

shaft and return to its holder.

3.20. TRAILER WITH FIFTH-WHEEL COUPLER &  DROP-LEG JACK

A fifth wheel coupler includes a flat load-bearing plate with a slot, and a mechanism inside the slot

that “grips” the kingpin.

We have installed a fifth wheel coupler that is suitable for the size and weight of the trailer. You

must provide a kingpin and kingpin plate that match the fifth wheel, and that is rated for the Gross

Vehicle Weight Rating (GVWR) of your trailer.

3.21. BEFORE ATTEMPTING THE TOW THE TRAILER

• Be sure the size and rating of the fifth wheel and kingpin match.

• Wipe the kingpin clean and inspect it visually and by feeling for flat spots, cracks and

pits. Check the condition of the kingpin mounting in the bed of the tow vehicle.

52

Coupling to the Tow Vehicle

^

^WARNING

A worn, bent, cracked or corroded kingpin can fail while towing, and may result in death or serious injury. Before coupling the trailer, inspect the kingpin and kingpin plate for wear, bending, cracks or corrosion; and replace worn or damaged kingpin.

• Be sure the fifth wheel mechanism operates freely.

• Lubricate the fifth wheel plate surface with a light coat of Lithium-base, waterproof grease.

• Be sure the fifth wheel and kingpin fasteners are tight and any welds are solid.

^^

^WARNING

A loose fifth wheel or kingpin can result in uncoupling, leading to death or serious injury. Be sure the fifth wheel and kingpin are tight before coupling the trailer.

Be sure the brake line, electrical line, and any other lines are clear of the coupling area.

Be sure the locks are open.

• If the tow vehicle is equipped with a tailgate, lower it.

• Block the trailer wheels, front and rear.

• Make certain that trailer fifth wheel plate is slightly above the kingpin plate on the tow

vehicle.

• Back tow vehicle up close to the trailer, centering the kingpin in the slot of the fifth

wheel.

• STOP before engaging the coupling.

AR

^WARNING

If the trailer drops during coupling, death or serious injury may result. There must be no one under the trailer or coupler before or during the coupling operation.

• Adjust the height of the trailer, using the jack, so that the fifth wheel plate just touches

the kingpin plate.

53

Coupling to the Tow Vehicle

• Slowly back up the tow vehicle, keeping the kingpin centered in the slot of the fifth

wheel. Continue backing up until the fifth wheel locks firmly on the kingpin.

• Visually check to confirm that the fifth wheel locks are properly locked onto the

kingpin.

• Attempt to pull forward as an initial test of the closing of the fifth wheel locks.

^^

^WARNING

An improperly coupled fifth wheel can come loose, resulting in death or serious injury. Do not tow the trailer until all of the visual checks have been performed: • Adjustment nut against fifth wheel. • Secondary lock behind yoke. • Fifth wheel against kingpin plate.

3.22. RAISE THE DROP-LEG JACK

A trailer having a fifth wheel coupler will be outfitted with one or two drop leg jacks for raising and

lowering the fifth wheel coupler. Because we use several drop leg jack mechanisms, the general

instructions below may vary slightly from the jack manufacturer’s instructions. If the trailer jack on

your trailer does not resemble the jack shown in the figures, follow the jack instructions provided

by the jack manufacturer. If you do not have these instructions, call Thuro-Bilt, Inc. at 541-670-8701

for a free copy.

Rotate the jack handle counter-clockwise. This will slowly retract the jack and transfer the

weight of the trailer tongue to the towing vehicle. When the drop leg base is no longer

resting on the ground, the towing vehicle hitch is holding all of the weight of the trailer

tongue. Continue retracting the jack to its fully retracted position.

Return the drop legs to their upper positions. The drop legs are held in the lowered position

with a plunger pin. Rotating the plunger pin while pulling it outward about ¾ inch will

cause it to come out of the engagement with the drop leg and the leg will rapidly raise

(see the “Releasing Drop Leg Mechanism” figure).

Raise the Tailgate

Pick up the trailer wheel blocks.

54

Coupling to the Tow Vehicle

^ CAUTION

The drop legs are heavily spring loaded in the lowered position. They will rapidly return to the upper position when released and can inflict serious bruises, scrapes or pinching. Keep your feet, shins and hands well clear of the drop legs and drop leg bases when releasing the drop legs. Always wear shoes or boots while performing this operation.

3.23. ATTACH & TEST THE BREAKAWAY BRAKE SYSTEM

If the coupler fails, a properly connected and working breakaway brake system will apply electric

brakes on the trailer.

The breakaway brake system includes a battery, a switch with a pullpin, and a breakaway brake

lanyard. Read and follow the instructions here as well as the instructions that have been prepared

by the breakaway brake controller manufacturer. If you do not have these instructions, call

Thuro-Bilt, Inc. at 541-670-8701.

The breakaway brake system may be fitted with a charging facility that draws power from the tow

vehicle. If the electrical system on your tow vehicle does not provide power to the breakaway

brake battery, you must periodically charge the battery to keep the breakaway brake system in

working order.

Visually inspect the breakaway brake system for broken parts.

Connect the pull pin cable to the tow vehicle (see “Breakaway Brake System” figure). Do not

connect to kingpin or its support.

To test the break-away battery remove the pullpin from the switch and attempt to pull the

trailer forward. You should feel the trailer resisting being towed, but the wheels will not

necessarily lock-up.

Immediately replace the pull pin. The breakaway brake system battery discharges rapidly

when the pull pin is removed.

55

Coupling to the Tow Vehicle

^WARNING

An ineffective breakaway brake system can result in a runaway trailer leading to death or serious injury, if the coupler fails. Connect the breakaway cable to the tow vehicle, and NOT to the kingpin or its support. Before towing the trailer, test the function of the breakaway brake system. If the breakaway brake system is not working, do not tow the trailer; have it serviced or repaired.

Do not tow the trailer with the breakaway brake system ON because the brake will overheat which

can result in permanent brake failure.

^W

^WARNING

Failure to replace the pullpin will prevent brakes from working, leading to loss of control, serious injury or death.

If you do not use your trailer for three or more months, or during winter months:

Store the battery indoors; and

Charge the battery every three months.

Replace the breakaway brake battery at intervals recommended by the battery manufacturer.

3.24. CONNECT THE ELECTRICAL CABLES

Connect the trailer lights to the tow vehicle's electrical system using the electrical connectors.

Check all lights for proper operation:

Clearance and Running Lights (Turn on tow vehicle headlights).

Brake Lights (Step on Tow vehicle brake pedal).

Turn Signals (Operate tow vehicle directional signal lever).

Check brakes for proper operation: While towing the trailer at less than 5 m.p.h., manually

operate the electric brake controller in the tow vehicle cab. You should feel the operation

of the trailer brakes.

^WARNING

56

Coupling to the Tow Vehicle

^W

^WARNING

Improper electrical connection between the tow vehicle and the trailer will result in inoperable lights and electric brakes, and can lead to collision. Before each tow: • Check that the taillights, brake lights and turn signals work • Check that the electric brakes work by operating the brake controller inside the tow vehicle.

3.25. UNCOUPLING THE FIFTH-WHEEL TRAILER WITH DROP-LEG JACK

Follow these steps to uncouple your fifth wheel hitch trailer from your tow vehicle

• Block trailer tires to prevent the trailer from rolling before jacking the trailer up.

• Disconnect the electrical connector.

• Disconnect the breakaway brake switch lanyard.

• If the tow vehicle has a tailgate, lower it.

• Make certain that ground surface below jack base will support trailer tongue load.

• Rotate the drop leg plunger pin handle so that the plunger pin is released from the

drop leg (see “Releasing Drop Leg Mechanism” figure).

• Push down on the drop leg base with your foot to place a drop leg to the desired

lowered position.

^ CAUTION

The drop legs are heavily spring loaded in the lowered position. They will rapidly return to the upper position when released and can inflict serious bruises, scrapes or pinching. Keep your feet, shins and hands well clear of the drop legs and drop leg bases when releasing the drop legs. Always wear shoes or boots while performing this operation.

• Rotate the plunger pin handle so that the plunger pin is attempting to engage the drop

leg (see “Releasing Drop Leg Mechanism” figure).

• Slowly raise your foot, permitting the drop leg to raise. The plunger pin will engage a

hole in the drop leg.

• Be sure the plunger pin is fully engaged. Push it in by hand if necessary. The bent

part of the plunger pin handle must be touching the plunger pin housing.

• If your trailer has two drop leg jacks, lower them both to the same level, following the

above instructions.

57

Coupling to the Tow Vehicle

^NOTICE

If the drop legs are not set at the same level, one of the drop leg jacks can be overloaded and can be damaged.

• Release the handle (or crank) from its holder and engage it with the jack shaft.

• Rotate the handle (or crank) clockwise to slowly extend the jack and transfer the

weight of the trailer tongue to the jack.

• On two speed jacks, pushing the handle shaft toward the gearbox can perform rapid

extension. This shifts the gearbox into a high speed mode.

• When the drop leg base contacts the ground, shift the gearbox into low speed mode

by pulling out on the handle shaft until it locks into low gear.

^NOTICE

Do not use high speed to lift the trailer, the drop leg jack mechanism can be damaged. High speed is used only to rapidly move the drop leg base into contact with the ground.

• Continue to extend the jack(s), making sure that the ground is providing stable and

level support for the trailer.

• Turn the crank two or three turns to take some of the weight of the coupling. Do not

raise the fifth wheel off the kingpin plate.

• After the jack(s) are extended enough to permit driving the tow vehicle away,

disengage the jack handle from its shaft and return it to its holder. Do NOT drive the

tow vehicle yet!

Open the fifth wheel locks by:

• pulling the release handle, or

• using a separate pipe release handle to engage the solid stud on the secondary

Lock.

Slowly drive the tow vehicle away from the trailer.

Raise the tow vehicle tailgate.

58

Loading the Trailer

4. LOADING THE TRAILER

Improper trailer loading causes many accidents and deaths. To safely load a trailer, you must

consider:

Overall load weight;

Load weight distribution;

Proper tongue weight; and

Securing the load properly.

To determine that you have loaded the trailer within its rating, you must consider the distribution of

weight, as well as the total weight of the trailer and its contents. The trailer axles carry most of the

total weight of the trailer and its contents (Gross Vehicle Weight, or “GVW”). The remainder of the

total weight is carried by the tow vehicle hitch. It is essential for safe towing that the trailer-tongue

and tow vehicle hitch carry the proper amount of the loaded trailer weight, otherwise the trailer can

develop an undesirable sway at towing speeds, or the rear of the towing vehicle can be

overloaded. Read the “Tongue Weight” section below.

The load distribution must be such that no component part of the trailer is loaded beyond its

rating. This means that you must consider the rating of the tires, wheels and axles. For tandem

and triple axle trailers, you must make sure that the front-to-rear load distribution does not result in

overloading any axle.

Towing stability also depends on keeping the center of gravity as low as possible. Load heavy

items on the floor and over the axles. When loading additional items, be sure to maintain even

side-to-side weight distribution and proper tongue weight. The total weight of the trailer and its

contents must never exceed the total weight rating of the trailer (Gross Vehicle Weight Rating, or

“GVWR”).

^

^WARNING

An overloaded trailer can result in loss of control of the trailer, leading to death or serious injury. Do not load a trailer so that the weight on any tire exceeds its rating. Do not exceed the trailer Gross Vehicle Weight Rating (GVWR) or the Gross Axle Weight Rating (GAWR).

4.1. TONGUE WEIGHT

It is critical to have a portion of the trailer load carried by the tow vehicle. That is, the trailer

tongue must exert a downward force on the hitch. This is necessary for two reasons. First, the

proper amount of tongue weight is necessary for the tow vehicle to be able to maintain control of

the tow vehicle/trailer system. If, for example, the tongue exerts an upward pull on the hitch,

instead of pushing down on it (because the trailer is overloaded behind its axle(s)), the rear wheel

59

Loading the Trailer

of the tow vehicle can lose traction or grip and cause loss of control. Also, even if there is some

weight on the tongue, but not enough weight on the tongue, the trailer can become unstable at

high speeds. Remember, the faster you go the more likely the trailer is to sway.

If, on the other hand, there is too much tongue weight, the tow vehicle is prone to jack-knife.

Furthermore, the front wheels of the tow vehicle can be too lightly loaded and cause loss of

steering control and traction, if the front wheels are driving.

In addition to tow vehicle control, tongue weight is necessary to insure that the trailer axle(s) do

not exceed their Gross Axle Weight Rating (GAWR).

The table below has “rules of thumb” for proper tongue weight.

In the table below, the second column notes the rule of thumb percentage of total weight of the

trailer plus its cargo (Gross Vehicle Weight, or “GVW”) that should appear on the tongue of the

trailer. For example, a trailer with a gooseneck hitch, with a loaded weight of 12,000 pounds,

should have 20-25% (of 12,000 pounds) on the tongue. That is, the example trailer would have

2,400 to 3,000 pounds on its tongue.

| Tongue Weight as a Percentage of Loaded Trailer Weight |  |
| --- | --- |
| Type of Hitch | Percentage |
| Ball Hitch (or Bumper Hitch) | 10–15%for large trailers 6-10% for smaller utility and cargo trailers 4-6% for boat trailers |
| Gooseneck Hitch & Fifth Wheel Hitch | 20–25% |

The numbers quoted above are for example purposes only and should be tailored to the

specific trailer. For questions regarding the actual percent of tongue weight for the trailer,

check with the manufacturer for specifics.

60

Loading the Trailer

^W

^WARNING

Improper tongue weight (load distribution) can result in loss of control of the trailer, leading to death or serious injury. Make certain that tongue weight is within the allowable range. Be sure to: • Distribute the load front-to-rear to provide proper tongue weight (see chart); • Distribute the load evenly, right and left, to avoid tire overload; and • Keep the center of gravity low.

4.2. CHECKING TONGUE WEIGHT

To check the tongue weight, the tow vehicle and trailer must be on level ground, as they will be

when the trailer is being towed.

For lighter trailers the recommended method of checking tongue weight is to use an accessory

called a “tongue weight scale.” If a tongue weight scale is not available from your dealer, call

Thuro-Bilt, Inc. at 541-670-8701 for assistance.

An alternate method of checking tongue weight involves the use of a bathroom scale. The loaded

trailer must be on a smooth and level surface, and you must block the trailer wheels, front and

rear.

^

^WARNING

An unrestrained trailer can fall off its support, resulting in serious injury or death. Before checking tongue weight, block trailer wheels, front and rear.

Raise the tongue of the trailer with the jack.

Place a bathroom scale on the ground, directly below the coupler.

Place a strong block support (such as a cement block) on the scale – note the scale reading

for the weight of the block support.

Lower the tongue until the coupler rests on the block support and the jack is ½ inch above the

ground.

The scale reading, minus the weight of the block support is the tongue weight.

If the tongue weight exceeds the capacity of a bathroom scale, you can use “leverage” to

divide the tongue weight between the bathroom scale and another support Raise the

tongue of the trailer with the jack.

61

Loading the Trailer

For heavier trailers it is easier to go to a truck stop where there is a “certified” scale. Pull only the

tow vehicle onto the scale and get the weight. This weight must be less than your tow vehicle’s

GVWR. Pull the trailer onto the scale and decouple it from the tow vehicle, leaving just the trailer

on the scale. Get a “ticket”, which lists the total trailer weight. Re-connect the trailer to your tow

vehicle and the drive the tow vehicle wheels off the scale, just leaving the trailer axles on the

scale. Get a second “ticket”, which lists the trailer’s axle weight. Simple subtract the axle weight

from the total weight to determine the hitch weight.

While you are at the scale, you should weigh the entire combination vehicle. This result should be

less than the Gross Combined Weight Rating (GCWR) for your towing vehicle. Some scales allow

you to get individual axle weights also. If this is possible, get the tow vehicles front and rear axle

weights to make sure they are in the same proportion as the tow vehicle alone, and that the rear

axle is not overloaded. This is the best way to check that a weight distribution (or load leveling)

hitch is adjusted properly, i.e., you have the proper number of chain links attached to the snap-up

brackets.

4.3. SECURING THE CARGO

Since the trailer “ride” can be bumpy and rough, you must secure your cargo so that it does not

shift while the trailer is being towed.

^

^WARNING

Shifting cargo can result in loss of control of the trailer, and can lead to death or serious injury. Tie down all loads with proper sized fasteners, ropes, straps, etc.

4.4. LOADING CARGO (ENCLOSED TRAILER)

Couple the trailer to the tow vehicle before loading. The tongue of a bumper pull trailer can rise

during loading, before the cargo is properly distributed. To measure the tongue weight, you will

have to uncouple the trailer after it is loaded.

Do not transport people, containers of hazardous substances, cans or containers of flammable

substances. However, fuel in the tank of an off-road vehicle, or a car or motorcycle, etc., may be

carried inside of your enclosed cargo trailer.

62

Loading the Trailer

^W

^WARNING

Do not transport people inside the trailer, even if it has living quarters. The transport of people puts their lives at risk and may be illegal.

^WARNI

^WARNING

Do not transport flammable, explosive, poisonous or other dangerous materials in your trailer. Exceptions: • Fuel in the tanks of vehicles that are being towed. • Fuel stored in proper containers used in trailer living quarters for cooking. • Fuel stored in the tank of an on-board generator.

4.5. PREPARING THE TRAILER FOR LOADING

Before loading cargo into your enclosed trailer, inspect the interior of the trailer.

Enclosed trailers may be fitted with “D”-ring hold-downs, and/or a track system that can be used to

secure the cargo. Inspect the “D”-rings and track system for looseness or signs of bending before

loading the cargo onto the trailer.

^WARNING

Damaged or loose “D”-rings can break, allowing cargo to become loose inside the trailer. Loose cargo can shift the center of gravity, and result in loss of control of the trailer. Inspect “D”-rings, and test them for looseness before loading cargo. Do not use a damaged or loose “D”-ring to secure cargo.

4.6. LOADING THE ENCLOSED TRAILER

Enclosed trailers may be fitted with a drop ramp door. The weight of the drop ramp door is

partially held by a spring and cable counterbalance assembly. If the spring and cable

counterbalance assembly is out of adjustment or worn out, it will not provide the expected

assistance for slow and careful lowering and raising of ramp.

63

Loading the Trailer

^WA

^WARNING

A spring and cable counterbalance can inflict serious injury if it breaks, or if incorrectly adjusted. Inspect the cable and cable ends each time the door is operated. Do not attempt to service the counterbalance. Take the trailer to your dealer for service.

Carefully lower the drop ramp to the ground.

Load the cargo up the drop ramp and into the trailer, with approximately 60% of the cargo in the

front half of the trailer. If the trailer has living quarters, the cargo area of your trailer will have

ventilation openings near the floor. Do not block these ventilation openings. These openings are

provided to exhaust potentially deadly fumes.

^W

^WARNING

Accumulation of hazardous fumes can cause death or serious injury. Do not block access to ventilation ports.

Secure the cargo to the trailer using appropriate straps, chains and tensioning devices.

Close the drop ramp door and secure the trailer door catch using a linchpin or other locking

device, so that the catch and door cannot open while the trailer is being towed.^WAR

^WARNING

If the door opens, your cargo may be ejected onto the road, resulting in death or serious injury to other drivers. Always secure the door latch after closing. Place a linchpin in the catch.

4.7. LOADING HORSES (HORSE TRAILER)

Couple the trailer to the tow vehicle before loading. This is essential for the bumper pull trailer

because the tongue of a bumper pull trailer can rise during loading, before the cargo is properly

distributed.

The cargo-carrying portion of a horse trailer is designed only for carrying horses. Do not transport

people, livestock, containers of hazardous substances, or containers of flammable substances.

64

Loading the Trailer

^WARNING

^WARNING

Do not transport people inside the trailer, even if it has living quarters. The transport of people puts their lives at risk and may be illegal.

^WARNIN

^WARNING

Do not carry “loose” livestock in your horse trailer. They can cause the trailer to become unstable and can result in loss of control. You must use a trailer designed to carry “loose” livestock.

Before loading a horse in your trailer, inspect the interior of the trailer. The interior of the trailer

must be smooth, and have no protruding objects. There should be no loose objects that could

move about and startle or injure the horse. Check the walls, floor, dividers, etc., for loose and

broken parts, welds, hinges, etc.

4.8. PREPARING THE HORSE TRAILER FOR LOADING

Open windows and vents to provide ventilation. Consider the weather and transport conditions

(i.e. on warm sunny days, maximum ventilation is required). Do not carry a horse without

providing ventilation, even in coldest weather. Ventilation is critical for the well being of your

horses. Know your horses and adjust ventilation for your horses’ comfort.

Be sure pivoting window latches are in a flush position, so they do not present a protrusion that

can injure your horse.

Tighten any loose or protruding screws in the walls.

Remove or secure loose objects, (i.e. butt bars, saddles, tack and equipment) so that items will

not move during towing.

Inspect for cracks at the welds on the divider hinges, and the welds on the tie rings. If you are

able to open any cracks in or near these welds by lifting the dividers or by twisting the tie rings,

have the weld repaired before loading your horses.

65

Loading the Trailer

^ CAUTION

The trailer interior may contain hazards to a horse that can result in its serious injury or death. Before loading a horse, inspect the trailer interior and adjust or repair all loose and protruding features such as handles, loose or broken parts of the trailer, etc. Before towing trailer: • Lock all stall dividers. • Be sure all saddles, tack and equipment, as well as horse(s), are prevented from being thrown about.

^WARNING

Improper weld repair will lead to early failure of the trailer structure and can cause serious injury or death. Do not repair cracked or broken welds unless you have the skills and equipment to make a proper repair. Have the welds repaired by your dealer.

4.9. LOADING THE HORSE TRAILER

The trailering of horses introduces many variables that are not present in the trailering of nonliving

cargo. Horses are prone to take flight when they feel threatened or pain. In the confines of

a trailer, the flight response can cause serious injury or death to a human handler. Even

experienced and docile horses can be frightened.

Horses must be slowly acclimated to trailering. Be sure the horse’s first trips are short trips, so

you can gauge its reaction. Some will take to the experience easily, but others will strongly

protest. You must act according to your horse’s demeanor.

66

Loading the Trailer

^WA

^WARNING

When a horse is frightened, it is capable of inflicting serious injury or death to a human handler. Know your horse’s temperament before attempting to trailer it. Handling a horse that is not trailer-acclimated may result in injury or death, or damage to your trailer. Do not haul an unbroken horse in this trailer. Horses must have a halter.

Open all stall dividers and lock them in their OPEN (against the wall) position.

If the trailer has living quarters, close and lock the door between the living quarters and the horse

area.

If the trailer is fitted with a drop ramp, carefully lower it to the ground.

If your trailer is fitted with swinging loading doors, open them fully and fasten them against the

side of the trailer using the door holdbacks.

Lead the horse into the trailer by a halter or lead rope. If the horse shows any signs of distress,

stop loading, and calm the horse.

^

^WARNING

Improper weight distribution of the horses in the trailer will result in an unstable trailer. Always load the first horse into the forward most stall.

Tie the horse to the trailer interior by fastening the quick connect or tying the lead rope to the tie

ring, or other facility provided on the trailer wall for attachment of the lead rope. A rule of thumb is

to leave about 18 inches of free rope between the attachment point on the trailer and the horse.

The layout of the horse trailer has been designed to safely contain your horse. The trailer is

equipped with stall dividers and tie rings to secure the horse. Restraining a horse without using a

combination of a tie-strap and stall divider may result in serious injury or death to the horse.

^ CAUTION

Failure to secure a horse using a tie strap may result in its serious injury or death.

Close and lock the stall divider.

67

Loading the Trailer

If additional horses are to be loaded, repeat steps 5-7 above for each horse – lead the horse,

secure the horse, close and lock the stall divider.

After the last horse has been loaded, lock any unused dividers in the CLOSED (across the trailer)

position.

Double check that each horse is tied to the trailer and each stall divider is LOCKED in the

CLOSED position.

If your trailer is fitted with a butt bar or butt strap to keep the horse away from the door, hook and

lock the butt bar in place.

Close the trailer. Remove the door holdbacks and swing the hinged doors to a closed position, or

raise the drop ramp.

Secure the trailer door catch with a linchpin or similar device, so that the catch and door cannot

open while the trailer is being towed.

If your trailer is fitted with feed doors, close and secure them.

A

^WARNING

If the door opens, your cargo may be ejected onto the road, resulting in death or serious injury to other drivers. Always secure the door latch after closing. Place a linchpin in the catch.

Check the horses after 5 to 10 miles or 10 minutes of towing, and then at least once per hour

thereafter. Open a feed door or other access and look for signs of stress, cuts, or injury. On long

trips it is recommended that horses be removed from the trailer every 6-10 hours for exercise,

food and watering.

^WAR

N

^WARNING

Horses may kick when back door is opened. Stay clear when opening back door.

4.10. LOADING LIVESTOCK (LIVESTOCK TRAILER)

Couple the trailer to the tow vehicle before loading. This is essential for the bumper pull trailer

because the tongue of a bumper pull trailer can rise during loading, before the cargo is properly

distributed.

The cargo-carrying portion of a livestock trailer is for carrying livestock (other than horses) only.

The livestock trailer does not have the equipment required for the safe transport of horses, e.g.

stall dividers, tie rings and a rubber floor mat. Do not transport people, containers of hazardous

substances, or containers of flammable substances.

68

Loading the Trailer

^W

^WARNING

Do not transport people inside the trailer, even if it has living quarters. The transport of people puts their lives at risk and may be illegal.

W

^WARNING

Do not transport flammable, explosive, poisonous or other dangerous materials in your trailer.

^ CAUTION

Hauling a horse in a livestock trailer may result in its serious injury or death. Do not carry a horse in a livestock trailer. Use a trailer designed to carry horses.

4.11. PREPARING THE LIVESTOCK TRAILER FOR LOADING

Before loading livestock in your livestock trailer, inspect the interior of the trailer. The interior of

the trailer must be smooth, and have no protruding objects, such as bolts, broken parts of trailer

interior, etc. A protruding object can injure your livestock.

Tighten any loose or protruding bolts in the walls.

Remove or secure loose objects, so no items will move during towing.

^ CAUTION

The interior space of a trailer may contain hazards that result in serious injury or death to trailered livestock. Inspect the interior of the trailer before loading livestock. • Adjust or repair all loose and protruding features. • All cargo and equipment, besides the livestock, must be prevented from being thrown about before towing trailer.

69

Loading the Trailer

4.12. LOADING THE LIVESTOCK TRAILER

The trailering of livestock introduces many variables that are not present in the trailering of nonliving

cargo. Livestock may resist being loaded into a trailer.

^W

^WARNING

Large animals are capable of inflicting serious injury or death to a human handler. Know your animals’ temperament before attempting to trailer them.

If the trailer is fitted with a drop ramp, carefully lower it to the ground.

With the trailer in position, open and secure the loading door (either swinging or roll-up).

Open and secure the interior gates as necessary.

Guide the livestock into the trailer.

Gate the livestock tightly to keep them from moving or falling during transportation.

Close the loading doors (either swinging or roll-up) and raise the drop ramp.

Secure the trailer door catch with a linchpin or similar device, so that the catch and door cannot

open while the trailer is being towed.

^W

^WARNING

If the door opens, your cargo may be ejected onto the road, resulting in death or serious injury to other drivers. Always secure the door latch after closing. Place a linchpin in the catch.

70

Checking the Trailer Before & During Each Tow

5. CHECKING THE TRAILER BEFORE & DURING EACH TOW

5.1. PRE-TOW CHECKLIST

Before towing, double-check all of these items: See “7.1 Inspection, Service & Maintenance Summary Charts,”

for more information.

• Tires, wheels and lug nuts (see the “Major Hazards” section starting on page 1 of this manual)

• Tire Pressure. Inflate tire on trailer and tow vehicle to the pressure stated on the VIN /

Certification label.

• Coupler secured and locked (see the “Coupling and Uncoupling the Trailer” section starting on

page 32 of this manual)

• Safety chains properly rigged to tow vehicle, not to hitch or ball (see the “Coupling to the Tow

Vehicle” chapter starting at Page 29 of this manual)

• Test of lights: Tail, Stop, and Turn Lights

• Test trailer brakes.

• Safety breakaway switch cable fastened to tow vehicle, not to safety chains (see the

“Coupling to the Tow Vehicle” chapter starting at Page 29 of this manual)

• Cargo properly loaded, balanced and tied down (see the “Loading the Trailer” chapter starting

at page 59 of this manual)

• Tongue weight and weight distribution set-up.

• Doors and gates latched and secured

• Fire extinguisher

• Flares and reflectors

5.2. MAKE REGULAR STOPS

After each 50 miles, or one hour of towing, stop and check the following items:

• Coupler secured

• Safety chains are fastened and not dragging

• Cargo secured

• Cargo door latched and secured

• Live Animal Cargo Checks 4.3.3.2

71

Breaking in a New Trailer

6. BREAKING IN A NEW TRAILER

6.1. RETIGHTEN LUG NUTS AT FIRST 10, 25 & 50 MILES

Wheel lugs can shift and settle quickly after being first assembled, and must be checked after the first 10, 25

and 50 miles of driving. Failure to perform this check may result in a wheel coming loose from the trailer,

causing a crash leading to death or serious injury.

^WARNING

Lug nuts are prone to loosen after initial installation, which can lead to death or serious injury. Check lug nuts for tightness on a new trailer or when wheel(s) have been remounted after the first 10, 25 and 50 miles of driving.

See Section 7.26 page 82 on Proper Torqing Technique.

6.2. ADJUST BRAKE SHOES AT FIRST 200 MILES

Brake shoes and drums experience a rapid initial wear. The brakes must be adjusted after the first 200 miles

of use, and each 3,000 miles thereafter. Some axles are fitted with a mechanism that will automatically adjust

the brake shoes when the trailer is “hard braked” from a rearward direction. Read your axle and brake manual

to see if your brakes adjust automatically. If you do not have the axle and brake manual, call Thuro-Bilt, Inc. at

541-670-8701 for a free copy.

A hard stop is used to:

• Confirm that the brakes work;

• Confirm that the trailer brakes are properly synchronized with the tow vehicle brakes using the

brake controller in the tow vehicle

• Adjust the brake shoes as necessary.

• For surge brakes check the Master cylinder reservoir for fluid.

If your trailer is not fitted with automatically adjusting brakes, the brakes will need to be manually adjusted. See

section 7.7, “Manually Adjusting Brake Shoes,” for instructions.

6.3. SYNCHRONIZING THE BRAKE SYSTEMS

Trailer brakes are designed to work in synchronization with the brakes on the tow vehicle. When the tow

vehicle and trailer braking systems are synchronized, both braking systems contribute to slowing, and the

tongue of the trailer will neither dive nor rise sharply.

^WARNING

If trailer and tow vehicle brakes do not work properly together, death or serious injury can occur. Road test the brakes in a safe area at no more than 30 m.p.h. before each tow.

To insure safe brake performance and synchronization, read and follow the axle/brake and the brake controller

manufacturers’ instructions. If you do not have these instructions, call Thuro-Bilt, Inc. at 541-670-8701

for a free copy.

TIRE PRESSURE

Check tire pressures on both the trailer and tow vehicle. Inflate to the maximum shown on the VIN /

Certification Label Inspection, Service & Maintenance.

72

7. Inspection, Service & Maintenance

7.1. INSPECTION, SERVICE & MAINTENANCE SUMMARY CHARTS

You must inspect, maintain and service your trailer regularly to insure safe and reliable operation. If you cannot

or are unsure how to perform the items listed here, have your dealer do them. Note: In addition to this manual,

also check the relevant component manufacturer's manual.

| Inspection and Service before Each Use |  |  |
| --- | --- | --- |
| Item | Item Inspection / Service | Manual Section Reference |
| Breakaway Brakes > Electric > Hydraulic | Check operation. Check fluid level. | Sections 3.10, 3.17, 3.23 Sections 7.8, 7.9, 7.10, 7.11 |
| Breakaway Battery | Fully charged, connections Clean. | Sections 3.10, 3.17, 3.23 |
| Brakes, all types | Check operation. | Section 6 |
| Shoes and Drums | Adjust. | Section 6 & Section 7.6 |
| Brakes, Hydraulic – Vacuum Actuated | Check gauge for proper vacuum of 18 In. Hg. (inches of mercury) | Sections 7.12, 7.13, 7.14, 7.15 |
| Coupler and Hitch Ball | Check for cracks, pits, and flats. Replace w/ball & coupler having trailer GVW Rating. Grease. Check locking device & replace. | Section 3.5 & Section 7.16 |
| Gooseneck Ball | Check for cracks, pits, and flats. Replace w/ball & coupler having trailer GVW Rating. Grease. Check locking device & replace when worn. | Section 3.13 Section 7.17 |
| Fifth Wheel & Kingpin | Check for cracks, Grease. Check locking device & replace when worn. | Section 3.25 Section 7.18 |
| Safety Chain(s) & Hooks | Check for wear and damage. | Sections 3.9, 3.16 |
| Tires | Check tire pressure when cold. Inflate as needed. | Sections 2 & Section 5 Section 7.22 |
| Wheels - Lug Nuts (Bolts) & Hub | Check for tightness. Tighten. For new and remounted wheels, check torque after first 10, 25 & 50 miles of driving and after any impact. | Section 5 & Section 6 Section 7.22 |

73

Inspection, Service & Maintenance

| Inspection and Service each 3 Months or 3,000 Miles |  |  |
| --- | --- | --- |
| Item | Inspection / Service | Manual Section Reference |
| Structure > Rubber mats and floor > Hinges, Doors and dividers | Remove mats. Wash both sides. Wash floor. Inspect. Repair or replace damaged, worn or broken parts. | Sections 4.5, 4.8, 4.11 Section 7.3 |

| Inspection and Service each 6 Months or 6,000 Miles |  |  |
| --- | --- | --- |
| Item | Inspection / Service | Manual Section Reference |
| Tires | Rotate @ 5,000 miles. | Section 2 & Section 7.22 |
| Brakes, electric > Magnets > Controller (in tow vehicle) | Check wear and current draw. Check power output (amperage) and modulation. | Sections 7.6, 7.7, 7.8, 7.9, 7.10, 7.11 See Controller Mfr’s Manual |
| Structure > Roof Vents > Windows | Clean dirt buildup, lubricate hinges and slides. | Section 7.3 |
| Tires | Inspect tread and sidewalls thoroughly. Replace tire when treads are worn, when sidewall has a bulge, or sidewall is worn. | Section 2 Section 7.22 |

| Inspection and Service Each Year or 12,000 Miles |  |  |
| --- | --- | --- |
| Item | Inspection / Service | Manual Section Reference |
| Brakes, all types > Shoes and drums | Check for scoring and wear. Replace per manufacturers Specifications. | Sections 7.6, 7.7, 7.8, 7.9, 7.10, 7.11, 7.12, 7.13, 7.14, 7.15 See Brake Mfr’s Manual |
| Jack, Drop-leg | Grease gears at top | See Jack Mfr’s Manual |
| Structure > Frame members > Welds > Slide-out | Inspect all frame members, bolts & rivets. Repair or replace damaged, worn or broken parts. Inspect all welds. Repair as Needed. Clean dirt build-up. Lubricate slides, shafts and gears. | Sections 7.3, 7.4 |
| Wheels > Sealed Bearings (Hubs) > UNSEALED Bearings (Hubs) > Rims | Check and confirm free running. Replace if not (sealed bearings are not serviceable). Disassemble / inspect / assemble and repack. Replace promptly if immersed in water. Inspect for cracks & dents. Replace as needed. | Sections 7.23, 7.24, 7.25, 7.26 See Axle Mfr’s Manual |
| Structure > Axle Attachment Bolts | Check BY DEALER | Section 7.3 |

74

Inspection, Service & Maintenance

7.2. INSPECTION & SERVICE INSTRUCTIONS

W^

^WARNING

Worn or broken suspension parts can cause loss of control and injury may result. Have trailer professionally inspected annually and after any impact.

To perform many of the inspection and maintenance activities, you must jack up the trailer. “Jacking Points for

All Trailers” figure indicates the general areas where jacks and jack stands may be applied.

When jacking and using jack stands, place them so as to clear wiring, brake lines, and suspension parts

(springs, torsion bars, etc.). Place jacks and jack stands inside of the perimeter strip on the supporting

structure to which the axles are attached.

Jacking Points for Trailer

^

^WARNING

Never crawl under your trailer unless it is on firm and level ground and resting on properly placed and secured jack stands.

7.3. TRAILER STRUCTURE

Because the trailer floor receives the most abuse, it will most likely corrode before any other part of the

structure. This is particularly true for horse and livestock trailers, having floors subjected to urine and manure.

The urine and manure are corrosive to the metal or wood flooring and other structural parts of the trailer.

75

Inspection, Service & Maintenance

Remove the rubber mats from the floor of the trailer, and wash them, at least every three months. Using a

power washer and a detergent solution, wash both sides of the rubber mat, as well as the floor and walls of the

trailer. Rinse the rubber mat and the trailer floor and walls. Be sure the rubber mat and trailer floor are

completely dry before replacing the rubber mat.

7.4. WELDS

All welds can crack or fail when subjected to heavy loads or movement of cargo that was not properly tied to

prevent movement. Any time that you know or suspect that the trailer has been subjected to heavy loads or

movement of cargo, immediately inspect the welds and fasteners for damage. To prevent severe damage to

your trailer, inspect all of the welds for cracks or failure at least once a year.

^

^WARNING

Improper weld repair will lead to early failure of the trailer structure and can cause serious injury or death. Do not repair cracked or broken welds unless you have the skills and equipment to make a proper repair. If not, have the welds repaired by your dealer.

7.5. DROP RAMP TORSION SPRINGS

If your trailer has a drop-ramp door, the weight of the door may be partially held by a torsion spring and a cable.

Stand to the side when opening the drop ramp. You could be hurt if you are behind the drop ramp and the

counterbalance does not work.

Inspect the cable and cable ends regularly for fraying and signs of loosening. If released, a torsion spring can

inflict serious injury.

The torsion spring and cable are not user serviceable. The torsion spring must be serviced by a person who is

trained in torsion spring safety.

7.6. BRAKE SHOES & DRUMS

Properly functioning brake shoes and drums are essential to ensure safety. You must have your dealer inspect

these components at least once per year, or each 12,000 miles.

The brake shoes must be adjusted after the first 200 miles of use, and each 3,000 miles thereafter. Most axles

are fitted with a brake mechanism that will automatically adjust the brake shoes when the trailer is “hard

braked” from a rearward direction. Read your axle and brake manual to see how to adjust your brakes. If you

do not have this manual, call Thuro-Bilt, Inc. at 541-670-8701 for a free copy.

7.7. MANUALLY ADJUSTING BRAKE SHOES

Some braking systems are not automatically adjusted by hard stopping. These brakes require manual

adjustment. The following steps apply to adjust most manually adjustable brakes. Read your axle and brake

manual to see how to adjust your brakes. If you do not have this manual, call Thuro-Bilt, Inc. at 541-670-8701

for a free copy.

76

Inspection, Service & Maintenance

• Jack up the trailer and secure it on adequate capacity jack stands.

• Be sure the wheel and brake drum rotate freely.

• Remove the adjusting-hole cover from the adjusting slot on the bottom of the brake backing plate.

• With a screwdriver or standard adjusting tool, rotate the starwheel of the adjuster assembly to expand

the brake shoes. Adjust the brake shoes out until the pressure of the linings against the drum makes

the wheel very difficult to turn. Note: Your trailer maybe equipped with drop spindle axles. See axle

manual for your axle type. You will need a modified adjusting tool for adjusting the brakes in these

axles. With drop spindle axles, a modified adjusting tool with about an 80 degree angle should be

used.

• Rotate the star wheel in the opposite direction until the wheel turns freely with a slight drag.

• Replace the adjusting-hole cover.

• Repeat the above procedure on all brakes.

• Lower the trailer to the ground.

7.8. BRAKES, ELECTRIC

Two different types of electric brakes may be present on the trailer: an emergency electric breakaway system,

which acts only if the trailer comes loose from the hitch and the breakaway pin is pulled. The other brake is an

electric braking system that acts whenever the brakes of the tow vehicle are applied.

7.9. BREAKAWAY BRAKE

This battery supplies the power to operate the trailer brakes if the trailer uncouples from the tow vehicle. Be

sure to check, maintain and replace the battery according to the battery manufacturer’ instructions.

This switch causes the breakaway battery to operate the electric brakes if the trailer uncouples from the tow

vehicle.

The pull cable for the pull pin is connected to the tow vehicle, and the switch is connected to the trailer. To

check for proper functioning of the switch, battery and brakes, you must pull the pin from the switch and confirm

that the brakes apply to each wheel. You can do this by trying to pull the trailer with the tow vehicle, after

pulling the pin. The trailer brakes may not lock, but you will notice that a greater force is needed to pull the

trailer.

^

^WARNING

If electric breakaway brakes do not operate when trailer is uncoupled from the tow vehicle, death or serious injury can occur. Check emergency breakaway brake system BEFORE each tow.

7.10. TOW VEHICLE OPERATED ELECTRIC BRAKES

The electric brakes that operate in conjunction with the tow vehicle brakes must be “synchronized” so that

braking is properly distributed to the tow vehicle brakes and the trailer brakes. For proper operation and

synchronization, read and follow the axle/brake and the brake controller manufacturers’ instructions. If you do

not have these instructions, call Thuro-Bilt, Inc. at 541-670-8701 for a free copy.

77

Inspection, Service & Maintenance

7.11. MAGNETS FOR ALL ELECTRIC BRAKES

To make certain an electrically-operated braking system will function properly, you must have your dealer

inspect the magnets at least once a year, or each 12,000 miles. See the brake manual for wear and current

inspection instructions.

7.12. BRAKES, HYDRAULIC (VACUUM, AIR or ELECTRIC OPERATED)

If your trailer has hydraulically-operated brakes, they function the same way the hydraulic brakes do on your

tow vehicle. The hydraulic braking system must be inspected by a dealer, at least as often as the brakes on

the tow vehicle, but no less than once per year. This inspection includes an assessment of the condition and

proper operation of the wheel cylinders, brake shoes, brake drums and hubs.

You must check the fluid level in the master cylinder reservoir at least every three months. If you tow your

trailer an average of 1,000 miles per month in a hot and dry environment, you must check the brake fluid level

once a month. The brake fluid reservoir is located on the tongue of the trailer or near the gooseneck. Fill with

DOT 4 brake fluid.

7.13. VACUUM-OPERATED HYDRAULIC

When towing a trailer, the vacuum gauge, which is located inside the cab of the tow vehicle, must indicate 18

In. Hg. (inches of mercury) or more at all times.

^WA

^WARNING

If the vacuum gauge in tow vehicle is not at or above 18 In. Hg. (inches of mercury), damage to the brake system will result and the brakes may become inoperable.

7.14. AIR PRESSURE-OPERATED HYDRAULIC

Air/hydraulic braking systems are typically used when the tow vehicle has a diesel engine. The tow vehicle has

an air compressor that routes the air to an air/hydraulic mechanism, which sends brake fluid to the wheel

cylinders.

The air pressure gauge in your tow vehicle indicates the current air pressure. See your tow vehicle manual for

the proper air pressure.

7.15. ELECTRICAL-OPERATED HYDRAULIC

Electric/hydraulic braking systems, which are mounted on the trailer, use a small electrically-driven pump to

generate hydraulic pressure, which operates the brake cylinders. Like electrical brakes, an electric/hydraulic

braking system is operated by an electrical signal from the tow vehicle.

7.16. COUPLER & BALL

The coupler on the trailer connects to the ball attached to the hitch on the tow vehicle. The coupler, ball and

hitch transfer the towing forces between the tow vehicle and the trailer. Before each tow, coat the ball with a

thin layer of automotive bearing grease to reduce wear and ensure proper operation; and check the locking

device that secures the coupler to the ball for proper operation.

See the coupler manufacturer’s manual for other inspection and maintenance activities. If you do not have this

manual, call Thuro-Bilt, Inc. at 541-670-8701 for a free copy.

78

Inspection, Service & Maintenance

If you see or feel evidence of wear, such as flat spots, deformations, pitting or corrosion, on the ball or coupler,

immediately have your dealer inspect them to determine the proper action to prevent possible failure of the ball

and coupler system. All bent or broken coupler parts must be replaced before towing the trailer.

The coupler handle lever must be able to rotate freely and automatically snap into the latched position. Oil the

pivot points, sliding surfaces, and spring ends with SAE 30W motor oil. Keep the ball pocket and latch

mechanism clean. Dirt or contamination can prevent proper operation of the latching mechanism.

When replacing a ball, the load rating must match or exceed the GVWR of the trailer.

7.17. GOOSENECK

The gooseneck receiver on the trailer connects to a hitch-mounted ball on the towing vehicle. The receiver,

ball and hitch transfer the towing forces between the tow vehicle and the trailer. Check the locking device that

secures the receiver to the ball for proper operation.

See the gooseneck ball receiver manufacturer’s manual for other inspection and maintenance activities. If you

do not have a manual for the receiver, call Thuro-Bilt, Inc. at 541-670-8701 for a free copy.

If you see or can feel evidence of wear, such as flat spots, pitting or corrosion, on the ball or receiver,

immediately have your dealer inspect them to determine the proper action to prevent possible failure of the ball

and receiver system.

When replacing a ball, the load rating must match or exceed the GVWR of the trailer.

7.18. FIFTH WHEEL KINGPIN

Before each tow, inspect the fifth wheel and kingpin for wear, and coat the contact surface of the fifth wheel

plate with water-resistant Lithium-base grease. If you see evidence of wear on the fifth wheel or kingpin,

immediately have your dealer inspect them to determine the proper action to prevent failure of the fifth wheel

and kingpin system.

See the manual prepared by the manufacturer of the fifth wheel and kingpin for other inspection and

maintenance activities.

7.19. LANDING LEG OR JACK

If a grease fitting is present, you must use a grease gun to lubricate the jack mechanism. Grease the gears in

the top of hand-cranked jacks once a year, by removing the top of the jack and pumping or hand packing

grease into the gears.

7.20. LIGHTS & SIGNALS

Before each tow, check the trailer taillights, stoplights, turn signals and any clearance lights for proper

operation.

^W

^WARNING

Improper operating taillights, stoplights and turn signals can cause collisions. Check all lights before each tow.

79

Inspection, Service & Maintenance

7.21. ACCESSORY BATTERY

Your trailer may be outfitted with an accessory battery that operates lighting, electric landing gear, slide-outs or

other accessories. An accessory battery may be kept charged either by the tow vehicle or by the generator or

shore power. See the manual for the accessory battery.

A disconnect switch may be provided to disconnect the accessory battery when you do not plan to be using the

trailer for an extended period, such as seasonal storage. If there is no disconnect switch, then remove the

cables from the battery terminals.

The accessory battery must be kept in a charged condition during storage. The battery could freeze and break

if it becomes discharged.

7.22. TIRES

Trailer tires may be worn out even though they still have plenty of tread left. This is because trailer tires have to

carry a lot of weight all the time, even when not in use. It is actually better for the tire to be rolling down the road

than to be idle. During use, the tire releases lubricants that are beneficial to tire life. Using the trailer tires often

also helps prevent flat spots from developing.

The main cause of tire failure is improper inflation. Check the cold tire inflation pressures at least once a week

for proper inflation levels. “Cold” means that the tires are at the same temperature as the surrounding air, such

as when the vehicle has been parked overnight. Wheel and tire manufacturers recommend adjusting the air

pressure to the trailer manufacturer’s recommended cold inflation pressure, in pounds per square inch (PSI)

stated on the vehicle’s Federal Certification Label or Tire Placard when the trailer is loaded to its gross vehicle

weight rating (GVWR). If the tires are inflated to less than the recommended inflation level or the GVWR of the

trailer is exceeded, the load carrying capacity of the tire could be dramatically affected. If the tires are inflated

more than the recommended inflation level, handling characteristics of the tow vehicle/trailer combination could

be affected. Refer to the owner’s manual or talk to your dealer or vehicle manufacturer if you have any

questions regarding proper inflation practices.

Tires can lose air over a period of time. In fact, tires can lose 1 to 3 PSI per month. This is because molecules

of air, under pressure, weave their way from the inside of the tire, through the rubber, to the outside. A drop in

tire pressure could cause the tire to become overloaded, leading to excessive heat buildup. If a trailer tire is

under-inflated, even for a short period of time, the tire could suffer internal damage.

High speed towing in hot conditions degrades trailer tires significantly. As heat builds up during driving, the

tire’s internal structure starts to breakdown, compromising the strength of the tire. It is recommended to drive at

moderate speeds.

Statistics indicate the average life of a trailer tire is about five years under normal use and maintenance

conditions. After three years, replacing the trailer tires with new ones should be considered, even if the tires

have adequate tread depth. Some experts claim that after five years, trailer tires are considered worn out and

should be replaced, even if they have had minimal or no use. This is such a general statement that it may not

apply in all cases. It is best to have your tires inspected by a tire supplier to determine if your tires need to be

replaced.

If you are storing your trailer for an extended period, make sure the tires are fully inflated to the maximum rated

pressure and that you store them in a cool, dry place, such as a garage. Use tire covers to protect the trailer

tires from the harsh effects of the sun.

80

Inspection, Service & Maintenance

^WARNING

Worn, damaged or under-inflated tires can cause loss of control, resulting in damage, serious injury and possibly death. Inspect tires before each tow.

7.23. WHEEL RIMS

If the trailer has been struck, or impacted, on or near the wheels, or if the trailer has struck a curb, inspect the

rims for damage (i.e. being out of round); and replace any damaged wheel. Inspect the wheels for damage

every year, even if no obvious impact has occurred.

7.24. WHEELS, BEARINGS & LUG NUTS

A loose, worn or damaged wheel bearing is the most common cause of brakes that grab.

To check your bearings, jack trailer and check wheels for side-to-side looseness. If the wheels are loose, or

spin with a wobble, the bearings must be serviced or replaced.

7.25. UNSEALED BEARINGS (HUBS)

If your trailer has unsealed axle bearings, they must be inspected and lubricated once a year or 12,000 miles to

insure safe operation of your trailer.

If a trailer wheel bearing is immersed in water, it must be replaced.

If your trailer has not been used for an extended amount of time, have the bearings inspected and packed more

frequently, at least every six months and prior to use.

Exploded view of Wheel Bearing

Follow the steps below to disassemble and service the UNSEALED wheel bearings.

• After removing the grease cap, cotter pin, spindle nut and spindle washer (items 7-10 in “Exploded

Wheel Bearing” figure), remove the hub and drum to inspect the bearings for wear and damage.

81

Inspection, Service & Maintenance

• Replace bearings that have flat spots on rollers, broken roller cages, rust or pitting. Always replace

bearings and cups in sets. The inner and outer bearings are to be replaced at the same time.

• Replace seals that have nicks, tears or wear.

• Lubricate the bearings with a high quality EP-2 automotive wheel bearing grease.

Every time the wheel hub is removed and the bearings are reassembled, follow the steps below to check the

wheel bearings for free running and adjust.

• Turn the hub slowly, by hand, while tightening the spindle nut, until you can no longer turn the hub by

hand.

• Loosen the spindle nut just until you are able to turn it (the spindle nut) by hand. Do not turn the hub

while the spindle nut is loose.

• Put a new cotter pin through the spindle nut and axle.

• Check the adjustments. Both the hub and the spindle nut should be able to move freely (the spindle

nut motion will be limited by the cotter pin).

7.26. LUG NUTS (BOLTS)

Being sure wheel mounting nuts (lug nuts) on trailer wheels are tight and properly torqued is an important

responsibility that trailer owners and users need to be familiar with and practice. Inadequate and/or

inappropriate wheel nut torque (tightness) is a major reason that lug nuts loosen in service. Loose lug nuts can

rapidly lead to a wheel separation with potentially serious safety consequences.

Lug nuts are prone to loosen right after a wheel is mounted to a hub. When driving on a new or remounted

wheel, check the lug nut tightness often during the first few hundred miles of the trailer’s use, especially after

the first 10, 25 and 50 miles of driving, before each tow, and at least twice per year thereafter.

^

^

^WARNING

Lug nuts are prone to loosen after initial installation, which can lead to death or serious injury. Check lug nuts for tightness on a new trailer or when wheel(s) have been remounted after the first 10, 25 and 50 miles of driving.

^

^WARNING

Metal creep between the wheel rim and lug nuts will cause rim to loosen and could result in a wheel coming off, leading to death or serious injury. Tighten lug nuts before each tow.

Tighten the lug nuts to the proper torque for the axle size on your trailer to prevent wheels from coming loose.

Use a torque wrench to tighten the fasteners. The only way to be certain you have checked the torque or

torqued the lug nuts to the proper torque is with a torque wrench. Four-way wrenches, ratchets, and similar

tools can be useful for short-term emergency repairs, but are not appropriate tools for adequately checking lug

nut torque. You must use a torque wrench to adequately indicate the torque that you are applying to the lug

nut. If you do not have a torque wrench, tighten the fasteners with a lug wrench as much as you can, then

have a service garage or dealer tighten the lug nuts to the proper torque. Over-tightening will result in breaking

the studs or permanently deforming the mounting stud holes in the wheels.

82

Inspection, Service & Maintenance

Keep a record of the date and approximate mileage when you check the lug nut torque. Note any lug nut that

has lost torque. Investigate the reason(s) if the lug nut torque is not maintained after more than one re-torque

application, because this indicates there is something wrong with the lug nuts, nut studs, wheels and/or hubs

and should be corrected.

Contact your dealer or vehicle manufacturer immediately if you experience any persistent lug nut loosening or

any other lug, wheel or axle problems.

In the event of a wheel separation incident, notify the vehicle manufacturer and dealer. Seek prompt

professional assistance in assessing the trailer and its gear, and retain, but don’t re-use involved lugs, wheels

and studs. Don’t repair or service the trailer yourself. Call a trained technician.

| Lug Nut Torque – Steel Wheels |  |  |
| --- | --- | --- |
| Axle Rating Pounds | Stud Size | Dry Torque Foot-pounds |
| 3,500 to 7,000 | ½ inch | 85lb -95lb |

Dry Torque Foot-pounds is determined by manufacturer.

83

Warning Stickers on the Trailer

8. WARNING STICKERS ON THE TRAILER

84

Warning Stickers on the Trailer

85

Warning Stickers on the Trailer

This Warning Sticker is placed on top of each fender. DO NOT STEP ON THE FENDERS, THEY ARE NOT A STEP.

Action is unsafe and could result of injury or damage to the trailer, which will not be covered under warranty.

^WARNING

DO NOT USE THE HANDLES ON THE DROP DOWN WINDOWS AS ROPE TIES. SEE PHOTO.

^NOTICE

MAKE SURE AND UNLATCH THE REAR DOOR HOLD BACK BEFORE ATTEMPTING TO CLOSE THE REAR DOOR. SEE PHOTOS.

LATCHED				        UNLATCHED

86